NOTICE

DISCLAIMER. The information contained in this publication is subject to constant review in the light of changing government requirements and regulations. No subscriber or other reader should act on the basis of any such information without referring to applicable laws and regulations and/or without taking appropriate professional advice. Although every effort has been made to ensure accuracy, the International Air Transport Association shall not be held responsible for any loss or damage caused by errors, omissions, misprints or misinterpretation of the contents hereof. Furthermore, the International Air Transport Association expressly disclaims any and all liability to any person or entity, whether a purchaser of this publication or not, in respect of anything done or omitted, and the consequences of anything done or omitted, by any such person or entity in reliance on the contents of this publication.

© International Air Transport Association. All Rights Reserved. No part of this publication may be reproduced, recast, reformatted or transmitted in any form by any means, electronic or mechanical, including photocopying, recording or any information storage and retrieval system, without the prior written permission from:

Senior Vice President

Safety & Flight Operations

International Air Transport Association

800 Place Victoria

P.O. Box 113

Montreal, Quebec

CANADA H4Z 1M1

ISSA Part II Checklist
ISSM Part II Edition 4

© 2024 International Air Transport Association. All rights reserved.

Montreal—Geneva

# Section 2 —  Flight Operations (FLT)

Applicability Section 2 addresses safety and security requirements for flight operations of eligible operators. To be eligible, an Operator must meet the following criteria: - Seaplane operations; - Commercial passenger and/or cargo operations; - One- or two-pilot operations; - IFR and/or VFR operations; - Aircraft below and above 5,700 Kg (12,566 lb) MTOW. The standards and recommended practices in Section 2 are applicable only to those seaplanes and amphibians that are of the type authorized in the Air Operator Certificate (AOC) or equivalent document, and are utilized in commercial passenger and/or cargo operations unless applicability is extended to encompass non-commercial operations as stated in a note immediately under the body of the provision. Individual provisions or sub-specifications within a provision that: - Do not begin with a conditional phrase are applicable unless determined otherwise by the Auditor. - Begin with a conditional phrase (“If the Operator...”) are applicable if the operator meets the condition(s) stated in the phrase.

General Guidance The definitions of technical terms used in this ISSM Section 2, as well as the list of abbreviations and acronyms, are found in the IATA Reference Manual for Audit Programs (IRM).

## 1 Management and Control

### 1.3 Accountability, Authorities and Responsibilities

 FLT 1.3.6

The Operator should assign responsibility to the pilot-in-command (PIC) for: - The safety of all crew members, passengers and/or cargo on board the aircraft when the doors are closed; - The operation and safety of the aircraft from the moment the aircraft is ready to move for the purpose of taking off until the moment it finally comes to rest at the end of the flight and the engine(s) are shut down; - Ensuring checklists are complied with. (GM) Note: Effective 1 September 2025, this Recommendation will be upgraded to a standard.

☐ Documented and Implemented (Conformity) ☐ Documented not Implemented (Observation) ☐ Implemented not Documented (Observation) ☐ Not Documented not Implemented (Observation) ☐ N/A

Auditor Comments Click or tap here to enter text.

Auditor Actions ☐ Identified/Assessed document that assign responsibilities to the PIC. ☐ Interviewed responsible manager(s) in flight operations. ☐ Interviewed flight/cabin crew members. ☐ Other Actions (Specify) Click or tap here to enter text.

Guidance The intent of this provision is to ensure that the specified responsibilities are assigned to the PIC and such assignment is evident in Operator policies or procedures. Specifications in item i) and ii) may be satisfied by policies documented in, or referenced in, the OM that assign responsibilities to the PIC in a manner consistent with regulations of the State and the intent of the provision. Slight variations in the wording of policies are permissible if the periods of responsibility as specified in each item are addressed by the operator's policies. For example, an operator could assign responsibility to the PIC for the safety of passengers from the time they board the aircraft until they deplane. Such policy would satisfy this provision because it exceeds the period of PIC responsibility as specified in this provision. The specification in item iii) may be satisfied by any policy or combination of policies that assign the responsibility for compliance with standard operating procedures to the PIC.

 FLT 1.3.7

The Operator should ensure, for the duration of each flight, one pilot is designated to act as PIC. (GM) Note: Effective 1 September 2025, this Recommendation will be upgraded to a standard.

☐ Documented and Implemented (Conformity) ☐ Documented not Implemented (Observation) ☐ Implemented not Documented (Observation) ☐ Not Documented not Implemented (Observation) ☐ N/A

Auditor Comments Click or tap here to enter text.

Auditor Actions ☐ Identified/Assessed document that describe flight crew composition and/or succession of command. ☐ Interviewed responsible manager(s) in flight operations. ☐ Interviewed flight crew members. ☐ Other Actions (Specify) Click or tap here to enter text.

Guidance The specification of this provision is satisfied if one pilot is designated to act as PIC, regardless of crew configuration or en route crew changes.

### 1.5 Provision of Resources

 FLT 1.5.3

The Operator should have a process to ensure candidates, prior to being employed as flight crew members, are screened for the purpose of determining if they possess the requisite certifications, skills, competencies and other attributes required by the Operator and/or State. Such process, as a minimum, shall include procedures for reviewing and/or assessing: - Technical and non-technical competencies and skills, to include interpersonal skills; - Aviation experience; - Credentials and licenses; - Medical fitness; - Security background; - Common language(s) fluency. (GM) Note: Effective 1 September 2025, this Recommendation will be upgraded to a standard.

☐ Documented and Implemented (Conformity) ☐ Documented not Implemented (Observation) ☐ Implemented not Documented (Observation) ☐ Not Documented not Implemented (Observation) ☐ N/A

Auditor Comments Click or tap here to enter text.

Auditor Actions ☐ Identified/Assessed the process/criteria used for pre-employment screening of flight crew member candidates. ☐ Interviewed responsible manager(s) in flight operations. ☐ Interviewed selected flight crew candidate screening records. ☐ Other Actions (Specify) Click or tap here to enter text.

Guidance The specification in: - Item i) refers to technical competencies and skills that will vary with the requirements of the position in which the flight crew member will be employed. For example, an ab initio pilot will not necessarily have flying skills but will possess other skills and/or attributes necessary to succeed in training. - Item iii) typically includes verification of authenticity of licenses. - Item iv) could be assessed by a flight operations management interview, Human Resource interview and/or the conduct of a psychological analysis. - Item vi) is applicable unless such check is performed or prohibited by the State. - Item vii) refers to aviation English language fluency (where required for Air Traffic Control (ATC) communications) and sufficient fluency in the designated common language(s) necessary for ensuring effective communication (see FLT 3.1.1). Specifications in item i) and ii) may be satisfied by policies documented in, or referenced in, the OM that assign responsibilities to the PIC in a manner consistent with regulations of the State and the intent of the provision. Slight variations in the wording of policies are permissible if the periods of responsibility as specified in each item are addressed by the operator's policies. For example, an operator could assign responsibility to the PIC for the safety of passengers from the time they board the aircraft until they deplane. Such policy would satisfy this provision because it exceeds the period of PIC responsibility as specified in this provision. The specification in item iii) may be satisfied by any policy or combination of policies that assign the responsibility for compliance with standard operating procedures to the PIC.

 FLT 1.5.4

The Operator should have a process for screening candidates for the position of PIC, to include, if applicable, ensuring a prerequisite minimum level of line experience that is acceptable to the Authority. (GM) Note: Effective 1 September 2025, this Recommendation will be upgraded to a standard.

☐ Documented and Implemented (Conformity) ☐ Documented not Implemented (Observation) ☐ Implemented not Documented (Observation) ☐ Not Documented not Implemented (Observation) ☐ N/A

Auditor Comments Click or tap here to enter text.

Auditor Actions ☐ Identified/Assessed the processes used for screening of candidates for the position of PIC. ☐ Interviewed responsible manager(s) in flight operations. ☐ Interviewed selected flight crew candidate screening records. ☐ Other Actions (Specify) Click or tap here to enter text.

Guidance The specifications of this provision refer to a screening process for direct hire or upgrade to PIC. Such screening occurs prior to a pilot being assigned duties as PIC and typically includes: - Training records review; - Management recommendations and/or review board; - Training department recommendations and/or review board; - Verification of minimum experience acceptable to the Authority; - Any other screening requirements in accordance with the needs of the operator or requirements of the Authority.

### 1.6 Documentation System

 FLT 1.6.6

The Operator should ensure documents that comprise the onboard library, as specified in Table 2.1, are carried on board the aircraft for each flight and located in a manner that provides for access by the flight crew. (GM) Note: Effective 1 September 2025, this Recommendation will be upgraded to a standard.

☐ Documented and Implemented (Conformity) ☐ Documented not Implemented (Observation) ☐ Implemented not Documented (Observation) ☐ Not Documented not Implemented (Observation) ☐ N/A

Auditor Comments Click or tap here to enter text.

Auditor Actions ☐ Identified/Assessed the document that describes the onboard library. ☐ Interviewed responsible manager(s) in flight operations. ☐ Interviewed person(s) responsible for flight operations documentation management/control process(es). ☐ Other Actions (Specify) Click or tap here to enter text.

Guidance Refer to Table 2.1 for specifications related to accessing performance calculations via telecom systems (e.g. ACARS) in lieu of onboard documentation.

### 1.7 Operations Manual

FLT 1.7.1

The Operator shall have an Operations Manual (OM) for the use of personnel in the flight operations organization, which may be issued in separate parts, and which contains or references the policies, procedures, checklists and other guidance or information necessary for compliance with applicable regulations, laws, rules and Operator standards. As a minimum, the OM shall be managed and controlled in accordance with ORG 2.5.1 and be in accordance with specifications contained in Table 2.2. (GM)

☐ Documented and Implemented (Conformity) ☐ Documented not Implemented (Finding) ☐ Implemented not Documented (Finding) ☐ Not Documented not Implemented (Finding) ☐ N/A

Auditor Comments Click or tap here to enter text.

Auditor Actions ☐ Identified/Assessed operational documents that comprise the OM. ☐ Identified external documents referenced in the OM that contain operational information used by flight crew. ☐ Interviewed responsible manager(s) in flight operations. ☐ Examined selected parts of OM (focus: contents in accordance with Table 2.2). ☐ Other Actions (Specify) Click or tap here to enter text.

Guidance The intent of this provision is to ensure the flight crew will find all information necessary to perform its functions within the OM, or within another document that is referenced in the OM. The OM is identified as a source of operational information approved or accepted for the purpose by the operator or the State. Guidance and procedures in the OM enable the flight crew to comply with the conditions and limitations specified in the AOC.

 FLT 1.7.4

The Operator should have a process to develop and establish procedures and checklists for use by the flight crew. Such process shall ensure: - Human factors principles are observed in the design of the OM, checklists and associated procedures; - The specific parts of the OM relevant to flight crew are clearly identified and defined; - If applicable, any differences from procedures and checklists provided by the manufacturer(s) are based on operational considerations. (GM) Note: Effective 1 September 2025, this Recommendation will be upgraded to a standard.

☐ Documented and Implemented (Conformity) ☐ Documented not Implemented (Observation) ☐ Implemented not Documented (Observation) ☐ Not Documented not Implemented (Observation) ☐ N/A

Auditor Comments Click or tap here to enter text.

Auditor Actions ☐ Identified/Assessed process used to develop flight crew checklists and procedures. ☐ Identified specific parts of OM relevant to flight crew. ☐ Interviewed responsible manager(s) in flight operations. ☐ Other Actions (Specify) Click or tap here to enter text.

Guidance Refer to the IRM for the definition of Human Factors Principles. The intent of this provision is to ensure procedures and checklists are developed in a manner that ensures they are useable, identifiable and consistent with manufacturer specifications. Any deviations from manufacturer procedures or checklists are typically based on operational concerns identified by the operator. Human factors principles in document design and checklist usage typically address the following: - Preparation of documentation in a useable format for information presentation, at the appropriate reading level and with the required degree of technical sophistication and clarity. - Improving user performance through the use of effective and consistent labels, symbols, colors, terms, acronyms, abbreviations, formats and data fields. - Ensuring the availability and usability of information to the user for specific tasks, when needed, and in a form that is directly usable. - Designing operational procedures for simplicity, consistency and ease of use. - Enabling operators to perceive and understand elements of the current situation and project them to future operational situations. - Minimizing the need for special or unique operator skills, abilities, tools or characteristics. - Assessing the net demands or impacts upon the physical, cognitive and decision-making resources of the operator, using objective and subjective performance measures. - The specification in item ii) ensures the relevant sections of the OM are clearly identified as the OM can, in some instances, include sections published for flight operations personnel other than flight crew. As such, all OM sections need not be provided to the flight crew (e.g., training syllabi are usually restricted to training/checking personnel).

### 1.8 Operations to or from Uncertified Aerodromes

FLT 1.8.1

If the Operator provides scheduled air service for the purpose of transporting persons between one or more uncertified water aerodromes, the Operator shall be authorized to do so in its Air Operator Certificate (AOC). (GM)

☐ Documented and Implemented (Conformity) ☐ Documented not Implemented (Finding) ☐ Implemented not Documented (Finding) ☐ Not Documented not Implemented (Finding) ☐ N/A

Auditor Comments Click or tap here to enter text.

Auditor Actions ☐ Identified/Assessed the documents that authorize the Operator to conduct scheduled air services with passengers utilizing uncertified water aerodromes. ☐ Interviewed responsible manager(s) in flight operations. ☐ Examined AOC (focus: information is current and supports scheduled passenger service to uncertified water aerodromes). ☐ Other Actions (Specify) Click or tap here to enter text.

Guidance Many water aerodromes are uncertified. Typically, state authorities require that scheduled passenger air services be conducted between certified aerodromes. The intent of this provision is to ensure seaplane operators that are utilizing uncertified water aerodromes for scheduled flights are authorized to do so within their Air Operator Certificate (AOC).

FLT 1.8.2

If the Operator conducts scheduled flights to or from uncertified water aerodromes, the Operator shall have a process for assessing their suitability in accordance with the criteria set forth in Table 2.6 and the shall include, as part of or separate from the route manual, a water aerodrome manual documenting: - The TORA, TODA, and LDA for principal or designated take-off and landing areas; - Any protected areas; - Depths of taxi channels and maneuvering areas; - Any visual aids for navigation such as wind socks, taxi channel markings and other maritime traffic markers; - Tide and current information, if applicable; - Description and/or images of docks, moorings, or other passenger transfer facilities. (GM)

☐ Documented and Implemented (Conformity) ☐ Documented not Implemented (Finding) ☐ Implemented not Documented (Finding) ☐ Not Documented not Implemented (Finding) ☐ N/A

Auditor Comments Click or tap here to enter text.

Auditor Actions ☐ Identified/Assessed the process for assessing and documenting uncertified water aerodromes. ☐ Interviewed responsible manager(s) in flight operations. ☐ Examined water aerodrome charts and documentation (focus: uncertified water aerodrome documentation detailing the criteria set out in Table 2.6). ☐ Other Actions (Specify) Click or tap here to enter text.

Guidance The intent of this provision is to ensure that, notwithstanding the water aerodrome certification and operating requirements of the state regulator, water aerodromes used regularly by seaplane operators are assessed to conform with recommended safety standards, and that documentation is available to pilots so that they can familiarize themselves with the challenges particular to the locations typically used as part of the operator's route structure. In accordance with the recommendations in ICAO Annex 14, section 1.4.2, “States should certify aerodromes open to public use.” ICAO Annex 6 section 4.1 also requires that ground/water facilities intended for use are adequate for the safe operation of the type of flight to be conducted. Because seaplane operators in some states utilize water aerodromes that are uncertified, the responsibility falls upon the operator to make these assurances. Water aerodrome environments are more dynamic than airport environments. Because they are generally not purpose-built as aviation infrastructure, they are often used by other vessels and beset by challenges posed by terrain, sea state, and obstacles. Additionally, unlike most airports, many water aerodromes are not certified and lack publications describing the operating details required by pilots to safely utilize them. The responsibility of the operator is to make this information available to the pilots that are operating at these locations. Water aerodromes are also prone to change. This is particularly the case for uncertified and unmonitored or unmaintained water aerodromes. In these scenarios, the state and safety of fixed docks or floating platforms can change or water hazards and obstacles can develop. In consideration of this, an operator might consider establishing a procedure which enables pilots who are using unmonitored water aerodromes to file status reports that log any changes which could affect their safety or usability. An online forum such as Slack is an effective tool to keep all pilots apprised of changing conditions. Expanded guidance on water aerodrome certification may be found in the following ICAO documents: APAC Regional Guidance On Requirements For The Design And Operations Of Water Aerodromes For Seaplane Operations and the APAC Water Aerodrome Working Group Meeting (APAC WAWG−WP04).

FLT 1.8.3

If the Operator is utilizing uncertified water aerodromes, the Operator should have a process for evaluating and categorizing these aerodromes and areas of intended use based on the operational and environmental challenges posed to flight crews at each location. The considerations for this operational assessment are summarized in Table 2.6. This system should be integrated into the operator's line qualifications system such that: - Line qualification should include training and LIFUS criteria for pilots to operate at water aerodromes based on the water aerodrome category; - Experience requirements should be established for pilots to operate at the various water aerodrome categories; - Pilot experience and training requirements should be set for pilots operating at unfamiliar water aerodromes that are not documented as part of the operator's regular route structure. (GM)

☐ Documented and Implemented (Conformity) ☐ Documented not Implemented (Observation) ☐ Implemented not Documented (Observation) ☐ Not Documented not Implemented (Observation) ☐ N/A

Auditor Comments Click or tap here to enter text.

Auditor Actions ☐ Identified/Assessed procedures for evaluating and categorizing regularly used water aerodromes. ☐ Interviewed responsible manager(s) in flight operations. ☐ Examined Route manual or other documentation pertaining to assessment of water aerodromes (focus: categorization elements consistent with specifications in Table 2.6). ☐ Examined selected flight crew member training/qualification records (focus: LIFUS and experience qualifications for water aerodrome categories and use of unfamiliar water aerodromes). ☐ Other Actions (Specify) Click or tap here to enter text.

Guidance Operating aircraft on the water differs significantly from operating to and from land-based airports. Water aerodromes can pose challenges to seaplane pilots that require flying and pilot decision-making skills which are developed through experience. By identifying the challenges associated with the individual water aerodromes in their route structure and evaluating the risks associated with them, the operator can thereby ensure that the most qualified pilots are assigned to those routes and locations where increased skill and experience is required as well as ensuring that pilots are familiarized and assessed for each location with LIFUS training. An example of an aerodrome classification system could include three categories: - Category 1 would apply to water aerodromes for which there are no performance limitations or obstacle considerations in the approach and departure paths or within the take-off and landing areas. - Category 2 would apply to water aerodromes that have obstacle considerations, that do not have dock facilities or that have non-standard dock/passenger transfer facilities, or to unfamiliar aerodromes that have not been assessed. - Category 3 would apply to water aerodromes that present aircraft performance limitations or uniquely challenging geographic or environmental conditions such as unprotected take-off and landing areas or inclement weather. Specific pilot experience, training and/or LIFUS requirements would typically be required before pilots are able to operate at airports other than the first category.

## 2 Training and Qualification

### 2.1 Training and Evaluation Program

#### General

FLT 2.1.1

The Operator shall have a training and evaluation program, approved or accepted by the Authority, that consists of ground and flight training and, when applicable, evaluations to ensure flight crew members are competent to perform assigned duties. The program shall address traditional and, if applicable, advanced (or alternative) training and qualification, and ensure training and evaluation is conducted for each type of aircraft in the fleet. Such program shall also, as a minimum, address: - Initial qualification; - Continuing qualification; - Re-qualification; - As applicable, aircraft transition or conversion; - Upgrade to PIC; - As applicable, other specialized training requirements, including those associated with operations authorized the AOC. (GM)

☐ Documented and Implemented (Conformity) ☐ Documented not Implemented (Finding) ☐ Implemented not Documented (Finding) ☐ Not Documented not Implemented (Finding) ☐ N/A

Auditor Comments Click or tap here to enter text.

Auditor Actions ☐ Identified/Assessed flight crew training/qualification program (focus: program includes each type of aircraft in the fleet). ☐ Interviewed responsible manager(s) in flight operations. ☐ Examined training/qualification course curriculum for selected aircraft types (focus: inclusion of applicable training/qualification courses for each aircraft type). ☐ Examined training/qualification records of selected flight crew members (focus: completion of applicable training/qualification courses). ☐ Other Actions (Specify) Click or tap here to enter text.

Guidance The intent of this provision is to ensure an operator's training program contains the elements necessary to ensure flight crew members are continuously competent to perform assigned duties. The initial qualification process provided to newly hired crew members includes company indoctrination and initial endorsement on company aircraft types. This presupposes that the newly hired crew member already holds a commercial flying license. Initial endorsement training may not be required as part of initial qualification if a newly hired crew member already holds a type endorsement acceptable to both the State and the operator. Company indoctrination training, however, is always considered a part of initial qualification. Continuing qualification includes recurrent or refresher training and also includes any training necessary to meet recency-of-experience requirements. Transition (conversion) training refers to an aircraft type qualification training and evaluation program for each type of aircraft in the fleet and is not required when an operator only utilizes one type of aircraft. Specialized training could include training on a specific type of new equipment (e.g., ACAS) or training for specific operations to meet requirements of the Authority. Training could be outsourced, in which case services typically range from simple dry lease of a training device to delegation of all training to an external organization (e.g., Authorized Flight Training School).

FLT 2.1.4

If the Operator uses distance learning and/or distance evaluation in the flight crew training and qualification program, the Operator shall ensure such training and/or evaluation is monitored in accordance with FLT 2.1.28 and, if required, is approved or accepted by the State. (GM)

☐ Documented and Implemented (Conformity) ☐ Documented not Implemented (Finding) ☐ Implemented not Documented (Finding) ☐ Not Documented not Implemented (Finding) ☐ N/A

Auditor Comments Click or tap here to enter text.

Auditor Actions ☐ Identified/Assessed regulatory approval, process for monitoring/continual improvement of distance learning in flight crew training/evaluation program. ☐ Interviewed responsible manager(s) in flight operations. ☐ Examined selected distance learning/qualification course development records (focus: monitoring/continual improvement). ☐ Other Actions (Specify) Click or tap here to enter text.

Guidance Refer to the IRM for the definition of Distance Learning. Distance learning refers to flight crew training or evaluation that is not conducted in a classroom or face-to-face with an instructor or evaluator, but rather is conducted through the use of distributed printed material or electronic media (e.g., Internet, compact disc, etc.).

#### Training Manual

FLT 2.1.10

The Operator shall have a Training Manual for the use of flight operations personnel, which may be issued in separate parts, that contains the details of all relevant training programs, policies, procedures, requirements and other guidance or information necessary to administer the Operator's Training Program and the Training Manual shall, as a minimum, be managed and controlled as specified in ORG 2.5.1 and be in accordance with specifications contained in Table 2.2. (GM)

☐ Documented and Implemented (Conformity) ☐ Documented not Implemented (Finding) ☐ Implemented not Documented (Finding) ☐ Not Documented not Implemented (Finding) ☐ N/A

Auditor Comments Click or tap here to enter text.

Auditor Actions ☐ Identified/Assessed flight crew training manual, regulatory approval, content in accordance with Table 2.2. ☐ Interviewed the responsible manager(s) in flight operations. ☐ Examined selected parts of training manual (focus: content includes policies/procedures/requirements, other guidance/information necessary to administer the training/evaluation program). ☐ Other Actions (Specify) Click or tap here to enter text.

Guidance The training manual typically applies to instructors, evaluators, line–check airmen, flight crew members, training schedulers, simulator operations personnel, administrative support personnel and other applicable flight operations personnel. The training manual may be split among several publications with the relevant parts made easily accessible to the appropriate personnel.

FLT 2.1.28

The Operator shall have processes for ensuring continual improvement of the flight crew training and evaluation program, to include, as a minimum, the monitoring, recording and evaluation of results of successful and unsuccessful flight crew evaluations. (GM)

☐ Documented and Implemented (Conformity) ☐ Documented not Implemented (Finding) ☐ Implemented not Documented (Finding) ☐ Not Documented not Implemented (Finding) ☐ N/A

Auditor Comments Click or tap here to enter text.

Auditor Actions ☐ Identified/Assessed processes for program monitoring, continual improvement of flight crew training/evaluation program. ☐ Interviewed responsible manager(s) in flight operations. ☐ Examined selected records of program monitoring (focus: improvements resulting from monitoring). ☐ Other Actions (Specify) Click or tap here to enter text.

Guidance Flight crew operational non-compliances, training deficiencies and evaluation trends (simulator, aircraft and line operations) are typically used by the training organization for trend analysis and program improvement. Grading scale criteria (e.g. numerical, letter grade) provides a means to accurately identify areas for improvement.

### 2.2 Training Elements

FLT 2.2.1

The Operator shall ensure flight crew members complete training in maritime navigation regulations and knowledge as part of their initial ground training program. This training shall address topics applicable to the operating environment, including: - Right-of-way rules for seaplane operations on the water, in accordance with the International Regulations for Preventing Collisions at Sea; - The purpose and identification of marine navigation aids such as buoys, beacons, lights, and sound signals; - The use of nautical charts; - The use of tide and current tables for operations in tidal environments, if applicable. (GM)

☐ Documented and Implemented (Conformity) ☐ Documented not Implemented (Finding) ☐ Implemented not Documented (Finding) ☐ Not Documented not Implemented (Finding) ☐ N/A

Auditor Comments Click or tap here to enter text.

Auditor Actions ☐ Identified/Assessed the requirement for training in maritime navigation regulations and knowledge. ☐ Interviewed the responsible manager(s) in flight operations. ☐ Examined selected initial course curricula/syllabi pertaining to maritime navigation regulations and knowledge. ☐ Examined selected flight crew member training/qualification records (focus: maritime navigation regulations and knowledge). ☐ Other Actions (Specify) Click or tap here to enter text.

Guidance The intent of this provision is to ensure that the Operator's training program addresses areas of knowledge required by seaplane pilots which are unique to operating aircraft in maritime environments. While operating on the water, seaplanes are considered vessels and are subject to the same international rules of maritime navigation as other watercraft. Seaplane pilots must be able to read nautical charts and safely navigate to and from take-off and landing areas while complying with marine traffic rules and markers. When beaching or docking in tidal areas, tidal rise and fall must be considered to safely prevent the aircraft from striking submerged obstacles and shoals or allowing the aircraft to come to rest on the sea bottom. Tides and currents can have a significant impact on take-off and landing areas with regards to water conditions and the navigability of taxi channels, therefore an understanding of tide and current tables is an important skill for seaplane pilots operating in tidal areas. Expanded guidance can be found in ICAO Annex 2 (3.2.6 Water Operations), in the FAA Seaplane, Skiplane, and Float/Ski Equipped Helicopter Operations Handbook: Chapter 1, in the FAA Aeronautical Information Manual (AIM): 7-6-8. Seaplane Safety.

FLT 2.2.2

The Operator shall ensure that flight crew members complete initial and recurrent training and evaluation in seaplane handling skills. This training shall minimum include demonstration of competence in: - Docking, beaching and mooring techniques; - Taxiing skills, including displacement, plow and step taxi techniques; - Sailing techniques: - Water handling techniques for adverse conditions such as taxiing and docking in high winds and rough seas. (GM)

☐ Documented and Implemented (Conformity) ☐ Documented not Implemented (Finding) ☐ Implemented not Documented (Finding) ☐ Not Documented not Implemented (Finding) ☐ N/A

Auditor Comments Click or tap here to enter text.

Auditor Actions ☐ Identified/Assessed the requirement for training/evaluation of seaplane water handling techniques in flight crew training/evaluation program. ☐ Interviewed the responsible manager(s) in flight operations. ☐ Examined selected initial/recurrent training/qualification course curricula/syllabi pertaining to water handling techniques. ☐ Examined selected flight crew member training/qualification records (focus: completion of training/evaluation of water handling techniques). ☐ Other Actions (Specify) Click or tap here to enter text.

Guidance Handling a seaplane on the water requires different skills and experience than those associated with operating landplanes in an airport environment. Marine environments are dynamic and seaplane pilots must contend with wind, waves, obstacles, and other vessels while operating on the water. Because of the variability of conditions that these environments pose, what can be a simple procedure one day can be very challenging the next. The intent of this provision is to ensure that seaplane operator training programs contain the necessary elements to impart and evaluate these essential skills to their pilots, and to check that they maintain competence through recency and recurrent training. Expanded guidance can be found in the TCCA Instructor Guide, Seaplane Rating and in the FAA Seaplane, Skiplane, and Float/Ski Equipped Helicopter Operations Handbook: Chapter 4.

FLT 2.2.3

The Operator shall ensure flight crew members complete initial and subsequently annual recurrent flight training and evaluation related to seaplane flying which includes, at minimum, the following skill sets: - Rough water and glassy water take-off and landing techniques; - Crosswind take-off and landing techniques; - Wind, water and landing area evaluation and decision making skills. (GM)

☐ Documented and Implemented (Conformity) ☐ Documented not Implemented (Finding) ☐ Implemented not Documented (Finding) ☐ Not Documented not Implemented (Finding) ☐ N/A

Auditor Comments Click or tap here to enter text.

Auditor Actions ☐ Identified/Assessed the requirement for flight training/evaluation of seaplane take-off and landing techniques and skills in flight crew training/evaluation program. ☐ Interviewed the responsible manager(s) in flight operations. ☐ Examined selected initial/recurrent training/qualification course curricula/syllabi pertaining to seaplane flying techniques and skills. ☐ Examined selected flight crew member training/qualification records (focus: completion of training/evaluation of seaplane flying techniques and skills). ☐ Other Actions (Specify) Click or tap here to enter text.

Guidance Because no two bodies of water are alike and the same body of water can change dramatically from day to day based on environmental conditions, a seaplane pilot must develop, through training and experience, skills which contribute to safe decision-making and execution of take-offs and landing conducted on the water. The pilot's ability to assess take-off and landing conditions, to make decisions based on those assessments, and to utilize take-off and landing techniques appropriate to the conditions is paramount to mitigating the risks posed by the variable conditions encountered during seaplane operations. Studies have shown that nearly 70% of seaplane accidents occur during the take-off and approach and landing phases of flight and that the severity and incidence of these accidents are reduced in correlation to pilot experience and training. Expanded guidance can be found in the TCCA Instructor Guide, Seaplane Rating (TP 12668) and the FAA Seaplane, Skiplane, and Float/Ski Equipped Helicopter Operations Handbook: Chapters 4 and 6. Refer to Aviation Safety Study SSA9301 for more information pertaining to seaplane accidents, the contributing factors, and the mitigating effects of pilot skills and training.

FLT 2.2.4

The Operator should ensure that all seaplane pilots receive initial and recurrent underwater egress training.

☐ Documented and Implemented (Conformity) ☐ Documented not Implemented (Observation) ☐ Implemented not Documented (Observation) ☐ Not Documented not Implemented (Observation) ☐ N/A

Auditor Comments Click or tap here to enter text.

Auditor Actions ☐ Identified/Assessed the requirement for completion of an underwater egress training course by seaplane pilots on and initial and recurrent basis. ☐ Interviewed responsible manager(s) in flight operations. ☐ Examined selected flight crew member training/qualification records (focus: completion of initial/recurrent underwater egress training course). ☐ Other Actions (Specify) Click or tap here to enter text.

Guidance Investigations of seaplane accidents indicate that pilots and passengers involved in crashes on water often survived the impact but were unable to evacuate the aircraft and subsequently drowned. Data gathered by the TSB in Canada indicate that over the last 20 years approximately 70% of fatalities resulting from seaplane crashes in water were attributed to drowning. Regulators in countries with established legacies of seaplane operations are moving towards mandatory egress training for flight crews. In Canada, beginning in March 2022, initial training and then recurrent training every three years will be mandatory. This training greatly increases pilots' chances of successfully egressing from an inverted seaplane after an accident on the water, and thereby increases their ability to help passengers. For Expanded guidance refer to SOR/2019-49: Regulations Amending the Canadian Aviation Regulations (Parts I, VI and VII—Seaplane Operations) and for the coming changes to the TCCA Canadian Aviation Regulations, sections 703.98 and 704.115. Although other regulatory authorities have not mandated a requirement for this training, accident investigations and safety studies have shown that underwater egress training contributes significantly to the safety of seaplane operations in general and should be more broadly applied. Refer to TSB Safety Recommendation A13-02.

 FLT 2.2.7

The Operator should ensure flight crew members complete Operator familiarization training prior to being assigned to duties in line operations. Such training shall ensure familiarity with: - Duties and responsibilities; - Relevant state regulations; - Authorized operations; - Relevant sections of the OM. (GM) Conformance Applicability. Specific to Aircraft type; Included in Initial/Transition/ Conversion Training; Included in Recurrent Training/Continuing Qualification; Conformance through AQP/ATQP/EBT. Yes*; Yes; No; No. *This training may be provided as a complete package included in a company indoctrination course or, if applicable, tailored to address requirements that are different from the individual's previous training.. Note: Effective 1 September 2025, this Recommendation will be upgraded to a standard.

☐ Documented and Implemented (Conformity) ☐ Documented not Implemented (Observation) ☐ Implemented not Documented (Observation) ☐ Not Documented not Implemented (Observation) ☐ N/A

Auditor Comments Click or tap here to enter text.

Auditor Actions ☐ Identified/Assessed initial training/qualification course curriculum/syllabus (focus: operator familiarization training, definition of subjects addressed). ☐ Examined selected flight crew member training/qualification records. ☐ Interviewed responsible manager(s) in flight operations. ☐ Other Actions (Specify) Click or tap here to enter text.

Guidance This provision and many of the ensuing flight crew training provisions contain a Conformance Applicability (CA) Table. Refer to the General Guidance at the beginning of this Subsection 2, Training and Qualification, for a detailed description of the CA Table. Training is applicable to all flight crew members. Many operators refer to this training course as Basic Company Indoctrination.

FLT 2.2.12

If the Operator transports dangerous goods as cargo, the Operator shall ensure flight crew members complete training and an evaluation in dangerous goods during initial ground training and subsequently once during recurrent training within the 24-month period from the previous training in dangerous goods.

☐ Documented and Implemented (Conformity) ☐ Documented not Implemented (Finding) ☐ Implemented not Documented (Finding) ☐ Not Documented not Implemented (Finding) ☐ N/A

Auditor Comments Click or tap here to enter text.

Auditor Actions ☐ Identified/Assessed requirement for training/evaluation in dangerous goods in flight crew training/evaluation program. ☐ Interviewed responsible manager(s) in flight operations. ☐ Examined selected initial/recurrent training/qualification course curricula/syllabi (focus: dangerous goods training/evaluation; definition of specific aspects/subjects addressed). ☐ Examined selected flight crew member training/qualification records (focus: completion of training/evaluation in dangerous goods in initial/recurrent training). ☐ Other Actions (Specify) Click or tap here to enter text.

Guidance Training and evaluation is applicable to all flight crew members. Recurrent training in dangerous goods is typically completed within a validity period that expires 24 months from the previous training to ensure knowledge is current, unless a shorter period is defined by a competent authority. However, when such recurrent training is completed within the final 3 months of the 24-month validity period, the new validity period may extend from the month on which the recurrent training was completed until 24 months from the expiry month of the current validity period. If such recurrent training is completed prior to the final three months (or 90 days) of the validity period, the new validity period would extend 24 months from the month the recurrent training was completed. The curriculum for dangerous goods training for flight crew members will typically address the following subject areas: - General philosophy; - Limitations; - List of dangerous goods; - Labeling and marking; - Recognition of undeclared dangerous goods; - Storage and loading procedures; - Pilot's notification; - Provisions for passengers and crew; - Emergency procedures. Guidance may be found in the IATA Dangerous Goods Regulations (DGR) 1.5, Table 1.5.A.

FLT 2.2.13

If the Operator does not transport dangerous goods as cargo, the Operator shall ensure flight crew members complete training and an evaluation in dangerous goods during initial ground training and subsequently once during recurrent training within the 24-month period from the previous training in dangerous goods.

☐ Documented and Implemented (Conformity) ☐ Documented not Implemented (Finding) ☐ Implemented not Documented (Finding) ☐ Not Documented not Implemented (Finding) ☐ N/A

Auditor Comments Click or tap here to enter text.

Auditor Actions ☐ Identified/Assessed requirement for training/evaluation in dangerous goods in flight crew training/evaluation program. ☐ Interviewed responsible manager(s) in flight operations. ☐ Examined selected initial/recurrent training/qualification course curricula/syllabi (focus: dangerous goods training/evaluation; definition of aspects/subjects addressed). ☐ Examined selected flight crew member training/qualification records (focus: completion of training/evaluation in dangerous goods in initial/recurrent training). ☐ Other Actions (Specify) Click or tap here to enter text.

Guidance Training and evaluation is applicable to all flight crew members. Recurrent training in dangerous goods is typically completed within a validity period that expires 24 months from the previous training to ensure knowledge is current, unless a shorter period is defined by a competent authority. However, when such recurrent training is completed within the final 3 months of the 24-month validity period, the new validity period may extend from the month on which the recurrent training was completed until 24 months from the expiry month of the current validity period. If such recurrent training is completed prior to the final three months (or 90 days) of the validity period, the new validity period would extend 24 months from the month the recurrent training was completed. The curriculum for dangerous goods training for flight crew members is commensurate with responsibilities and will typically address: - General philosophy; - Limitations; - Labeling and marking; - Recognition of undeclared dangerous goods; - Provisions for passengers and crew; - Emergency procedures. Guidance may be found in DGR 1.5, Table 1.5.B.

FLT 2.2.16

The Operator shall ensure flight crew members complete training and an evaluation in subjects associated with adverse weather and/or environmental conditions during initial ground training and subsequently during recurrent training once every three (3) calendar years. - De-/anti-icing policies and procedures, if applicable to the operating environment; - Thunderstorm avoidance; - Meteorological systems associated with marginal VFR and IFR conditions; - The use of weather forecasting and reporting resources; - VFR weather minimums; - VFR weather decision making. (GM) Note: Item i) is applicable if the Operator conducts flights from any airport when conditions are conducive to ground aircraft icing.

☐ Documented and Implemented (Conformity) ☐ Documented not Implemented (Finding) ☐ Implemented not Documented (Finding) ☐ Not Documented not Implemented (Finding) ☐ N/A

Auditor Comments Click or tap here to enter text.

Auditor Actions ☐ Identified/Assessed requirement for training/evaluation in adverse weather/environmental conditions in flight crew training/evaluation program. ☐ Interviewed responsible manager(s) in flight operations. ☐ Examined selected initial/recurrent training/qualification course curricula/syllabi (focus: training/evaluation in adverse weather/environmental conditions; definition of aspects/subjects addressed). ☐ Examined selected flight crew member training/qualification records (focus: completion of training/evaluation in adverse weather/environmental conditions in initial/recurrent training). ☐ Other Actions (Specify) Click or tap here to enter text.

Guidance Training and evaluation is applicable to all flight crew members. The intent of this provision is to ensure flight crew members receive recurrent training and an evaluation in the subjects associated with the adverse weather or environmental conditions they may encounter in operations.

 FLT 2.2.20

If the Operator conducts flights into areas where English is the primary language of Air Traffic Control (ATC) and whose duties include communication with ATC, the Operator should require flight crew members to complete an evaluation during initial ground training to demonstrate a sufficient level of English language proficiency that will ensure effective communication during the performance of such duties. (GM) Note: Effective 1 September 2025, this Recommendation will be upgraded to a standard.

☐ Documented and Implemented (Conformity) ☐ Documented not Implemented (Observation) ☐ Implemented not Documented (Observation) ☐ Not Documented not Implemented (Observation) ☐ N/A

Auditor Comments Click or tap here to enter text.

Auditor Actions ☐ Identified/Assessed requirement for English language evaluation for flight crew members that will operate flights/communicate with ATC in areas where the primary language of ATC is English. ☐ Examined selected flight crew member training/qualification records (focus: completion of demonstration of English language proficiency). ☐ Examined selected flight crew member training/qualification records (focus: demonstration of English language proficiency level necessary for effective ATC communications). ☐ Interviewed responsible manager(s) in flight operations. ☐ Other Actions (Specify) Click or tap here to enter text.

Guidance The intent of this provision is to ensure a pilot who is required to communicate with air traffic control in English demonstrates a sufficient level of English language proficiency to ensure effective communication during the performance of duties. Such evaluation applies to each operating member of the flight crew, as required by the AFM, whose duties require communication in English with ATC. English proficiency requirements do not apply to flight engineers or flight navigators unless their duties include air/ground communication in English. A State requirement, as part of flight crew licensing, for an individual to demonstrate expert English language proficiency may be used to satisfy the specifications of this provision.

FLT 2.2.26

The Operator shall ensure flight crew members complete training in normal and non-normal procedures and maneuvers during initial training and subsequently during recurrent training once every 12 months. - Pilot Monitoring (PM), Pilot Flying (PF) and other flight crew division of duties (task sharing), if applicable; - If applicable, positive transfer of aircraft control; - Consistent checklist philosophy; - Emphasis on a prioritization of tasks (e.g. “aviate, navigate, communicate”); - Proper use of all levels of flight automation. (GM)

☐ Documented and Implemented (Conformity) ☐ Documented not Implemented (Finding) ☐ Implemented not Documented (Finding) ☐ Not Documented not Implemented (Finding) ☐ N/A

Auditor Comments Click or tap here to enter text.

Auditor Actions ☐ Identified/Assessed requirement for training in normal/non-normal procedures/maneuvers in flight crew training/evaluation program. ☐ Interviewed responsible manager(s) in flight operations. ☐ Examined selected initial/recurrent training/qualification course curricula/syllabi (focus: training in normal/non-normal procedures/maneuvers; definition of specific elements/subjects addressed). ☐ Examined selected flight crew member training/qualification records (focus: completion of initial/recurrent training in the specified normal/non-normal procedures/maneuvers). ☐ Other Actions (Specify) Click or tap here to enter text.

Guidance Refer to the IRM for the definitions of Pilot Flying (PF) and Pilot Monitoring (PM). Training is applicable to all flight crew members. The intent of this provision is to set a training interval for normal and non-normal procedures, and additionally to ensure the training manual, curricula, lesson plans, or other guidance associated with such training addresses the specifications in items (i) through (v). Division of flight crew duties, transfer of aircraft control, checklist use and prioritization of tasks are in accordance with the operator's policy for task sharing and as specified in FLT 3.11.18. Elements of training may be accomplished as part of ground, simulator, aircraft or line training. The term Pilot Monitoring (PM) has the same meaning as the term Pilot Not Flying (PNF) for the purpose of applying the specifications of this provision to two-pilot operations. The specification in item iv) refers to the following prioritization of tasks during any normal or abnormal situation or maneuver: - Aviate: fly the aircraft in accordance with restrictions and limitations set forth in the OM; - Navigate: guide the aircraft along the intended or appropriate route; - Communicate: verbalize intentions to other crew members and ATC, as applicable. The term “abnormal” is used to describe a condition or situation (e.g. abnormal airframe vibration, abnormal landing configuration). The terms “normal” and “non-normal/emergency” typically refer to AOM checklists, procedures and/or maneuvers. The term “non-normal” includes AOM emergency checklists and/or procedures (i.e. an emergency procedure is a subset of non-normal). The terms can also be used to describe an event, situation or operation that would be addressed by normal or non-normal/emergency procedures or checklists. When used in this manner, the terms may be separated by forward slash marks (e.g. normal/non-normal/emergency). The term “emergency” used alone refers to declarations and non-AOM procedures.

FLT 2.2.27

The Operator shall ensure flight crew members complete training and, when applicable, an evaluation, that includes a demonstration of competence in normal and non-normal procedures and maneuvers, to include, as a minimum, rejected takeoff, emergency evacuation, engine failure. Such training and, when applicable, evaluation shall be accomplished during initial training and subsequently during recurrent training once every 12 months. (GM)

☐ Documented and Implemented (Conformity) ☐ Documented not Implemented (Finding) ☐ Implemented not Documented (Finding) ☐ Not Documented not Implemented (Finding) ☐ N/A

Auditor Comments Click or tap here to enter text.

Auditor Actions ☐ Identified/Assessed requirement for training/evaluation including a demonstration of competence in normal/non-normal procedures/maneuvers in flight crew training/evaluation program. ☐ Interviewed responsible manager(s) in flight operations. ☐ Examined selected initial/recurrent training/qualification course curricula/syllabi (focus: training/evaluation in specified normal/non-normal procedures/maneuvers). ☐ Examined selected flight crew member training/qualification records (focus: completion of initial/recurrent training/evaluation in the specified normal/non-normal procedures/maneuvers). ☐ Other Actions (Specify) Click or tap here to enter text.

Guidance The intent of this provision is to define the basic initial and subsequent recurrent training and evaluation cycles that ensure flight crew members are competent to perform normal and non-normal procedures and maneuvers. It is understood that competence in all potential normal and non-normal procedures may not be demonstrated annually but in accordance with a schedule that is acceptable to the Authority. Training and, when applicable, a demonstration of competence in specified normal and non-normal procedures and maneuvers is applicable to all pilot crew members. Training and, when applicable, evaluation is to be accomplished as part of ground, simulator/aircraft and line training. Line training is in normal procedures/maneuvers only. Such evaluation of competence in the normal and non-normal procedures and maneuvers specified is applicable when such procedures and/or maneuvers are stipulated by the operator and/or State in conjunction with State-approved or State-accepted training courses that require a method of evaluation. Such courses typically include: - Type qualification; - Transition (conversion); - Upgrade to PIC; - Re-qualification; - Recurrent training. Operators that conduct training flights and cannot safely train/evaluate a non-normal procedure or maneuver in an aircraft or in a representative flight training device may demonstrate an alternative means of conformance.

 FLT 2.2.28

The Operator should ensure flight crew members complete practical training exercises: - In the use of emergency and safety equipment required to be on board the aircraft; - That address emergency evacuation and coordination among flight crew members and, as applicable, cabin crew members and/or supernumeraries required for the safety of operations. (GM) Note: Effective 1 September 2025, this Recommendation will be upgraded to a standard.

☐ Documented and Implemented (Conformity) ☐ Documented not Implemented (Observation) ☐ Implemented not Documented (Observation) ☐ Not Documented not Implemented (Observation) ☐ N/A

Auditor Comments Click or tap here to enter text.

Auditor Actions ☐ Identified/Assessed philosophy/requirements for preparing flight crew members for an evaluation in flight crew training/evaluation program. ☐ Examined guidance for instructors/evaluators (focus: methodology for providing information to flight crew members in preparation for an evaluation). ☐ Interviewed responsible manager(s) in flight operations. ☐ Other Actions (Specify) Click or tap here to enter text.

Guidance The specification of this provision is not intended to preclude flight crews from knowing the city pairs to be flown or the general maneuver requirements prior to the evaluation; however, flight crews would typically not be provided with the exact evaluation scenario. Operators that conduct training flights in an aircraft may divulge as much information about the intended training/evaluation as is necessary to ensure the safety of the planned operation.

FLT 2.2.30

The Operator shall ensure flight crew members complete training in CRM skills, which may be accomplished as part of simulator, aircraft and/or line training, as applicable. Such training shall be completed during initial training and subsequently during recurrent training once every 12 months. (GM)

☐ Documented and Implemented (Conformity) ☐ Documented not Implemented (Finding) ☐ Implemented not Documented (Finding) ☐ Not Documented not Implemented (Finding) ☐ N/A

Auditor Comments Click or tap here to enter text.

Auditor Actions ☐ Identified/Assessed requirements for training in CRM skills in flight crew training/evaluation program. ☐ Interviewed responsible manager(s) in flight operations. ☐ Examined selected initial/recurrent training/qualification course curricula/syllabi (focus: inclusion of CRM training in simulator/aircraft or during line flight training). ☐ Examined selected flight crew member training/qualification records (focus: completion of initial/recurrent CRM training). ☐ Other Actions (Specify) Click or tap here to enter text.

Guidance Training is applicable to all flight crew members. This specification is intended to ensure CRM skills are emphasized during and integrated into simulator or aircraft training, as applicable, and line training.

FLT 2.2.31

If the Operator utilizes single-pilot aircraft, the Operator shall ensure any flight crew members acting as single-pilot complete training in Single-Pilot Resource Management (SRM) skills, which may be accomplished as part of ground, simulator, aircraft and/or line training, as applicable. Such training shall be completed during initial training and subsequently during recurrent training once every 12 months. (GM)

☐ Documented and Implemented (Conformity) ☐ Documented not Implemented (Finding) ☐ Implemented not Documented (Finding) ☐ Not Documented not Implemented (Finding) ☐ N/A

Auditor Comments Click or tap here to enter text.

Auditor Actions ☐ Identified/Assessed requirements for training in SRM skills in flight crew training/evaluation Program. ☐ Interviewed responsible manager(s) in flight operations. ☐ Examined selected initial/recurrent training/qualification course curricula/syllabi (focus: inclusion of SRM training in ground, simulator/aircraft or during line flight training). ☐ Examined selected flight crew member training/qualification records (focus: completion of initial/recurrent SRM training). ☐ Other Actions (Specify) Click or tap here to enter text.

Guidance Training is applicable to flight crew members that are operating seaplanes single-pilot. The intent of this provision is to ensure that Single-Pilot Resource Management (SRM) concepts and skills are integrated into either the Operator's ground or flight training program(s) for single-pilot crew. SRM is a safety philosophy that uses the concepts of CRM to develop a set of skills and principles adapted to single-pilot operations. SRM is defined as the practice of managing all the resources available in a single-pilot cockpit to ensure safety of flight. These resources and skills include hazard recognition, risk management, decision-making skills, as well as Controlled Flight Into Terrain (CFIT) avoidance, and situational awareness. Expanded guidance can be found in Chapter 6 of the FAA Risk Management Handbook.

FLT 2.2.35

The Operator shall ensure that all flight crew members receive initial and annual CFIT awareness ground training. The training content shall focus on CFIT risks and avoidance strategies related to VFR operations and shall include, at minimum, the following content: - VFR weather minimums; - Minimum altitudes for the operator's route structure; - The use of weather forecasting and reporting resources; - Pilot Decision Making (PDM) skills and strategies; - GPS training, if applicable; - The risks associated with VFR flight in marginal VMC or into IMC conditions; - Causes and case studies of VFR CFIT accidents; - The risks associated with low altitude flight, including terrain, obstacles, blind canyons, stalls, and aircraft performance limitations; - CFIT avoidance and recovery techniques. (GM)

☐ Documented and Implemented (Conformity) ☐ Documented not Implemented (Finding) ☐ Implemented not Documented (Finding) ☐ Not Documented not Implemented (Finding) ☐ N/A

Auditor Comments Click or tap here to enter text.

Auditor Actions ☐ Identified/Assessed requirement for CFIT training in flight crew training program. ☐ Interviewed responsible manager(s) in flight operations. ☐ Examined selected initial/recurrent training course and curricula/syllabi (focus: suitability of CFIT training to VFR seaplane operations). ☐ Examined selected flight crew member training records (focus: completion of initial/recurrent training in CFIT awareness and avoidance). ☐ Other Actions (Specify) Click or tap here to enter text.

Guidance CFIT accidents present a disproportionate risk to seaplane operators due to the nature of VFR flight operations and the low-altitude flight regimes typically associated with seaplane operating conditions. Additionally, many state regulators do not require all seaplanes to be equipped with GPWS systems. The intent of this provision is to mitigate the elevated level of CFIT risk that operators are exposed to by requiring regular, comprehensive, and suitable CFIT awareness training for seaplane crews that is customized to the operating environment, and that focuses on VFR CFIT risks and avoidance strategies. Expanded guidance can be found in AC 61-134: General Aviation Controlled Flight Into Terrain Awareness. Additionally, Aviation Investigation Report A08P0353 provides a case study of the CFIT risks VFR seaplane operators are exposed to, as well as recommended pilot training related to weather, Pilot Decision Making (PDM), and GPS use.

 FLT 2.2.42

If the Operator transports passengers or supernumeraries, the Operator should ensure flight crew members complete security training as approved or accepted by the State, and in accordance with the Operator's security training program as specified in SEC 2.1.1 Flight crew security training shall address the following subject areas: - Determination of the seriousness of the occurrence; - Crew communication and coordination; - Policy and procedures associated with flight deck access; - Appropriate self-defense responses; - Use of non-lethal protective devices assigned to crew members for use as authorized by the State; - Understanding the behavior of terrorists so as to facilitate the ability to cope with hijacker behavior and passenger responses; - Situational training exercises regarding various threat conditions; - Flight deck procedures to protect the aircraft; - Aircraft search procedures; - As practicable, guidance on least-risk bomb locations. (GM) Conformance Applicability. Specific to Aircraft type; Included in Initial/Transition/ Conversion Training; Included in Recurrent Training/Continuing Qualification; Conformance through AQP/ATQP/EBT. Yes*; Yes; Yes (every 36 months); Yes. *This training may be provided as a complete package or, if applicable, tailored to address aircraft type or crew position requirements that are different from the individual's previous flight crew security training.. Note: . Flight crew members shall complete initial security training prior to being assigned to operational duties. Note: . The specifications of this provision are applicable to flight crew members used on board an aircraft during commercial and/or non-commercial operations. Note: . Effective 1 September 2025, this Recommendation will be upgraded to a standard.

☐ Documented and Implemented (Conformity) ☐ Documented not Implemented (Observation) ☐ Implemented not Documented (Observation) ☐ Not Documented not Implemented (Observation) ☐ N/A

Auditor Comments Click or tap here to enter text.

Auditor Actions ☐ Identified/Assessed flight crew security training program. ☐ Examined selected initial/recurrent training/qualification course curricula/syllabi. ☐ Examined selected flight crew training/qualification records. ☐ Interviewed responsible manager(s) in flight operations. ☐ Other Actions (Specify) Click or tap here to enter text.

Guidance Refer to the IRM for the definitions of Air Operator Security Program (AOSP) and Non-Lethal Protective Device. Flight crew members are directly involved in the implementation of security measures and thereby require an awareness of obligations to the Security Program of the operator. Crew security training would normally be in accordance with applicable regulations and/or the civil aviation security program of the State, and where no regulatory guidance exists, in accordance with the policy of the operator. Security training for flight crew members typically focuses on the need for the flight crew to maintain control of the flight deck. Specific subject areas included in recurrent security training are typically identified and derived from an analysis of actual or likely situations or trends experienced during line operations. Fight deck access as specified in item (iii) would typically include persons authorized for flight deck access as well as procedures for flight deck entry/exit. Flight crew training in self-defense responses as specified in item (iv) typically focuses on ensuring the security of the flight deck and takes into consideration relevant operational factors (e.g. type of operation, phase of flight, aircraft type/configuration, responses by cabin crew members or, if applicable, supernumeraries). Training as specified in item (vi) typically addresses topics or tactics as appropriate for the operator that might be associated with or could be used to facilitate crew-passenger reaction to or interaction with hijackers (e.g. conflict management, use of passive or non-passive cooperation, understanding Stockholm Syndrome, identification of and response to hijacker types/motives). Training exercises as specified in item (vii) are typically interactive in nature, and scenarios or situations (e.g. bomb threat, hijacking, unruly passenger) may be presented using various accepted training methods (e.g. live role playing, table top, computer-based training). Training as specified in item (x) is applicable to aircraft types that have designated least-risk bomb locations. Least-risk bomb locations are typically not identified on all-cargo aircraft. Refer to General Guidance at the beginning of this Subsection 2, Training and Qualification, for explanatory information regarding traditional training program requirements.

 FLT 2.2.43

If the Operator conducts passenger flights without cabin crew, the Operator should ensure flight crew members, complete training and demonstrate competence in the performance of any assigned duties and functions related to passenger cabin safety. Conformance Applicability. Specific to Aircraft type; Included in Initial/Transition/ Conversion Training; Included in Recurrent Training/Continuing Qualification; Conformance through AQP/ATQP/EBT. Yes*; Yes; Yes (every 36 months); Yes. *This training may be provided as a complete package or, if applicable, tailored to address aircraft type or crew position requirements that are different from the individual's previous training in duties and functions related to passenger cabin safety.. Note: Effective 1 September 2025, this Recommendation will be upgraded to a standard.

☐ Documented and Implemented (Conformity) ☐ Documented not Implemented (Observation) ☐ Implemented not Documented (Observation) ☐ Not Documented not Implemented (Observation) ☐ N/A

Auditor Comments Click or tap here to enter text.

Auditor Actions ☐ Identified/Assessed requirement for flight crew training in the performance of assigned duties/functions related to passenger cabin safety. ☐ Examined selected initial/recurrent training/qualification course curricula/syllabi. ☐ Examined selected flight crew training/qualification records. ☐ Interviewed responsible manager(s) in flight operations. ☐ Other Actions (Specify) Click or tap here to enter text.

### 2.3 Line Qualification

FLT 2.3.1

The Operator shall have a line qualification program consisting of line training and, where applicable, evaluations, approved or accepted by the State, which ensures flight crew members are qualified to operate in areas, on routes or route segments and into the airports or water aerodromes to be used in operations for the Operator. Such program shall: - Be published in the Training Manual or equivalent documents; - Ensure each pilot flight crew member has adequate knowledge of the elements specified in Table 2.5, as applicable to the areas, routes and route segments of intended operation; - Specify qualification requirements for operations in all areas, on all routes or route segments, and into all airports or water aerodromes and landing areas of intended use; - Ensure line training and evaluation for each pilot crew member is completed during initial qualification; - Ensure line training and evaluation is completed prior to a pilot crew member being used as a PIC in operations; (GM) - A Line Flying Under Supervision (LIFUS) or line qualification program shall set minimum standards for the required block time, sectors, and take-offs and landings on floats for seaplane PICs new to aircraft type or new to the operating environment. (GM)

☐ Documented and Implemented (Conformity) ☐ Documented not Implemented (Finding) ☐ Implemented not Documented (Finding) ☐ Not Documented not Implemented (Finding) ☐ N/A

Auditor Comments Click or tap here to enter text.

Auditor Actions ☐ Identified/Assessed flight crew line qualification training/evaluation program, approved/accepted by the State, specifies qualification requirements for operations associated with areas/routes/route segments/airports/water aerodromes used in operations. ☐ Interviewed responsible manager(s) in flight operations. ☐ Examined flight crew line qualification initial/recurrent curricula/syllabi (focus: line training/evaluation in areas/airports/water aerodromes of operations; program elements consistent with specifications in Table 2.5). ☐ Examined selected flight crew member training/qualification records (focus: completion of initial/recurrent line qualification training/evaluation). ☐ Other Actions (Specify) Click or tap here to enter text.

Guidance The intent of this provision is to ensure flight crew members are qualified to conduct routine operations within each theater of operation as defined by the operator. It does not address the additional and specialized knowledge. This specification in item v) and vi) applies to all candidates for the position of PIC, to include SIC upgrade candidates and pilots hired directly into PIC positions in operations for the operator. The training and evaluation specified in this provision is accomplished by pilot flight crew members as part of; ground training, simulator/aircraft training or line training.

FLT 2.3.5

The Operator should have a line check program to be conducted upon completion of initial flight training and LIFUS and then on a recurrent basis every 12 months thereafter, or as required by the state regulator, with specific emphasis on seaplane flying skills and pilot decision making. (GM)

☐ Documented and Implemented (Conformity) ☐ Documented not Implemented (Observation) ☐ Implemented not Documented (Observation) ☐ Not Documented not Implemented (Observation) ☐ N/A

Auditor Comments Click or tap here to enter text.

Auditor Actions ☐ Identified/Assessed requirement for flight crew line checks. ☐ Interviewed responsible manager(s) in flight operations. ☐ Examined flight crew line qualification initial/recurrent curricula/syllabi (focus: seaplane elements of line check program). ☐ Examined selected flight crew member training/qualification records (focus: completion of initial/recurrent line check evaluation). ☐ Other Actions (Specify) Click or tap here to enter text.

Guidance Initial and recurrent line checks for seaplane operators are not a requirement of all state regulators. However, the intent of this provision is to ensure that operators are regularly evaluating the flying and decision-making skills specific to seaplane flying which pilots must demonstrate during line operations. Some examples of these skills include: - Weather assessment; - Preflight inspection; - Docking; - Taxiing; - Inspection and judgement of take-off and landing areas; - Assessment of wind and water conditions for take-off and landing; - VFR en-route procedures and navigation; - Take-off and landing in variable wind and water conditions.

FLT 2.3.6

If the Operator utilizes both seaplanes and landplanes in their fleet, the Operator shall set pilot recency requirements that ensure pilots have completed, within the preceding 90 days, at least three take-offs and landings on each undercarriage type before carrying passengers. For pilots flying both amphibians and aircraft equipped with straight floats, the recency requirement shall be maintained on, at minimum, the amphibious aircraft. (GM)

☐ Documented and Implemented (Conformity) ☐ Documented not Implemented (Finding) ☐ Implemented not Documented (Finding) ☐ Not Documented not Implemented (Finding) ☐ N/A

Auditor Comments Click or tap here to enter text.

Auditor Actions ☐ Identified/Assessed recency requirements for pilots on each type of undercarriage flown. ☐ Interviewed responsible manager(s) in flight operations. ☐ Examined selected flight crew member training/qualification records (focus: completion of recency for take-offs and landings on each type of undercarriage and type flown). ☐ Other Actions (Specify) Click or tap here to enter text.

Guidance The intent of this provision is to ensure that pilots who are flying aircraft with different landing gear configurations maintain recency on floats. Additionally, due to the risk associated with gear down landings on water or gear up landings on land, pilots of amphibians who also fly seaplanes without retractable landing gear should complete the recency requirement on the amphibian, that being the most complex undercarriage type. Although most state regulators have recency requirements for take-offs and landings, these requirements generally only apply to the aircraft type flown, not the landing gear configuration. Studies have shown that nearly 70% of seaplane accidents occur within the take-off and landing stages of flight, and that pilot seaplane experience markedly reduces the incidence of those accidents. Take-off and landing recency requirements conducted on wheels do not adequately account for the unique skills required to operate a seaplane during those critical phases of flight. For expanded guidance refer to Aviation Safety Study SSA9301: A Safety Study Of piloting Skills, Abilities, And Knowledge In Seaplane Operations.

## 3 Line Operations

### 3.1 Common Language

 FLT 3.1.1

The Operator should ensure the designation of a common language(s) for use by all flight crew members for communication: - On the flight deck during line operations; - If the Operator conducts passenger flights with cabin crew, between the flight crew and cabin crew during line operations; - During flight crew training and evaluation activities. (GM) Note: Effective 1 September 2025, this Recommendation will be upgraded to a standard.

☐ Documented and Implemented (Conformity) ☐ Documented not Implemented (Observation) ☐ Implemented not Documented (Observation) ☐ Not Documented not Implemented (Observation) ☐ N/A

Auditor Comments Click or tap here to enter text.

Auditor Actions ☐ Identified/Assessed requirement for use of common language(s) by flight/cabin crew members. ☐ Interviewed selected flight/cabin crew members. ☐ Interviewed responsible manager(s) in flight operations. ☐ Other Actions (Specify) Click or tap here to enter text.

Guidance More than one common reference language might be designated. Communication in the designated common language is applicable to all flight crew members, including foreign nationals and expatriates used as flight crew members, instructors or evaluators by the operator. The operator is expected to be in compliance with the common language requirements of the State (e.g. mandatory for operations, a condition for employment or a condition for airman certification), if such requirements exist. If no State requirements exist, the operator is expected to designate an appropriate common operational language for use by flight crew members, as specified in this provision. The existence (and application) of a State common language requirement that satisfies the specifications of this provision relieves the operator of such a designation in operational documentation.

### 3.3 Flight Crew Qualifications

FLT 3.3.1

The Operator should have a policy that sets minimums for seaplane experience requirements which ensure flight crew are hired, trained, and upgraded according to the following standards: - PIC of single-engine aircraft with MTOW less than 5,000 pounds (2,270 kg): 100 hours float experience; - PIC of single-engine aircraft with MTOW of greater than or equal to 5,000 pounds (2,270 kg): 800 TT and 400 on floats; - PIC of single-engine aircraft authorized to carry more than nine passengers: 2000 TT, 500 PIC on floats; - PIC of multi-engine aircraft with MTOW of 12,500 pounds (5,670 kg) or more: 3000 TT, 250 PIC, 200 on type, 750 on floats. (GM)

☐ Documented and Implemented (Conformity) ☐ Documented not Implemented (Observation) ☐ Implemented not Documented (Observation) ☐ Not Documented not Implemented (Observation) ☐ N/A

Auditor Comments Click or tap here to enter text.

Auditor Actions ☐ Identified/Assessed policy for seaplane experience requirements in Operations Manual (OM) or other applicable document. ☐ Interviewed responsible manager(s) in flight operations. ☐ Examined selected flight crew member training/qualification records (focus: seaplane flight experience). ☐ Other Actions (Specify) Click or tap here to enter text.

Guidance The safety of seaplane operations is greatly enhanced by ensuring that pilots have accumulated seaplane flight experience. The skills, knowledge, and decision making inherent to operating seaplanes are unique and cannot be developed flying landplanes in typical runway to runway operations. A significant percentage of seaplane accidents are attributable to pilot error or contributing factors associated with pilot skill level, abilities, and knowledge. Studies have shown that the incidence of seaplane accidents decrease with an increase in pilot seaplane experience by mitigating causal factors such as selecting unsuitable take-off and landing areas, incorrectly assessing water and wind conditions, and loss of control during take-off and landing due to inadequate skills. Refer to Aviation Safety Study SSA9301: A Safety Study Of Piloting Skills, Abilities, And Knowledge In Seaplane Operations.

 FLT 3.3.2

The Operator should specify the composition and required number of flight crew members taking into account the type of aircraft, flight crew qualification requirements and flight/duty time limitations. Note: Effective 1 September 2025, this Recommendation will be upgraded to a standard.

☐ Documented and Implemented (Conformity) ☐ Documented not Implemented (Observation) ☐ Implemented not Documented (Observation) ☐ Not Documented not Implemented (Observation) ☐ N/A

Auditor Comments Click or tap here to enter text.

Auditor Actions ☐ Identified/Assessed requirement/methodology for determining flight crew composition/number of crew members based on aircraft type/crew qualification/flight duty time limitation. ☐ Examined selected flight crew pairing records. ☐ Interviewed responsible manager(s) in flight operations. ☐ Observed flight crew scheduling operations. ☐ Other Actions (Specify) Click or tap here to enter text.

Guidance The intent of this provision is to ensure flight crews are composed of the flight crew members appropriate for the aircraft type and planned operation. As applicable to an operator, crew composition requirements would typically also address the use of relief pilots and/or augmented crews.

FLT 3.3.4

The Operator shall ensure flight crew members will not operate an aircraft unless issued a medical assessment in accordance with requirements of the State; such assessment shall not be valid for a period greater than 12 calendar months. (GM) Note: If authorized by the State, it is permissible to extend the validity beyond 12 months (to preserve the original expiry date) when the medical assessment is renewed up to 45 days prior to its expiry date.

☐ Documented and Implemented (Conformity) ☐ Documented not Implemented (Finding) ☐ Implemented not Documented (Finding) ☐ Not Documented not Implemented (Finding) ☐ N/A

Auditor Comments Click or tap here to enter text.

Auditor Actions ☐ Identified/Assessed requirement for flight crew members to have valid medical assessment in accordance with requirements of the State, maximum 12 months validity. ☐ Identified/Assessed tracking/scheduling processes that prevent flight crew members from assignment to flight duty without valid medical assessment. ☐ Interviewed responsible manager(s) in flight operations. ☐ Examined selected flight crew training/qualification records (focus: existence of valid medical assessment). ☐ Other Actions (Specify) Click or tap here to enter text.

Guidance Requirements of the State and/or an applicable authority that are associated with medical classifications, aircraft types, flight crew positions and/or licensing could require a more restrictive assessment interval than specified in this provision. An applicable authority is one that has jurisdiction over international operations conducted by an operator over the high seas or the territory of a state that is other than the State of the Operator.

 FLT 3.3.7

The Operator should have a process to ensure flight crew member recency-of-experience requirements are satisfied as follows: - A pilot does not act as PIC or SIC of an aircraft unless either: - On the same type or variant of aircraft within the preceding 90 days (120 days if under the supervision of an instructor or evaluator), that pilot has operated the flight controls during at least three takeoffs and landings in the aircraft type or in a flight simulator approved for the purpose by the appropriate authority, or On the same type or variant of aircraft within a time period acceptable to the State and applicable authorities, that pilot has operated the flight controls during the number of takeoffs and landings in the aircraft type or in a flight simulator approved for the purpose by the appropriate authority, necessary to conform to a defined recency of experience schedule approved or accepted by the State and applicable authorities. - On the same type or variant of aircraft within a time period acceptable to the State and applicable authorities, that pilot has operated the flight controls during the number of takeoffs and landings in the aircraft type or in a flight simulator approved for the purpose by the appropriate authority, necessary to conform to a defined recency of experience schedule approved or accepted by the State and applicable authorities. - A pilot does not act in the capacity of a cruise relief pilot unless, within the preceding 90 days, that pilot has either: - Operated as PIC, SIC or cruise relief pilot on the same type or variant of aircraft, or - Completed flying skill refresher training to include normal, abnormal and emergency procedures specific to cruise flight on the same type of aircraft or in a flight simulator approved for the purpose, and has practiced approach and landing procedures, where the approach and landing procedure practice may be performed as the PM. - flight engineer does not perform duties in an aircraft unless either: - Within the preceding 6 months, that individual has had at least 50 hours of flight time as a flight engineer on that aircraft type aircraft, or - Within the preceding 90 days, that individual has operated as a flight engineer on board that aircraft type or in a simulator of the aircraft type. - A flight navigator or radio operator does not perform duties in an aircraft unless recency-of-experience requirements of the Operator and the State have been satisfied. - If a flight crew member does not satisfy recency-of-experience requirements in accordance with i), ii), iii) or iv), such flight crew member completes re-qualification in accordance with the Operator's training and evaluation program. (GM) Note: Effective 1 September 2025, this Recommendation will be upgraded to a standard.

☐ Documented and Implemented (Conformity) ☐ Documented not Implemented (Observation) ☐ Implemented not Documented (Observation) ☐ Not Documented not Implemented (Observation) ☐ N/A

Auditor Comments Click or tap here to enter text.

Auditor Actions ☐ Identified/Assessed tracking/scheduling processes that prevent flight crew members from flight duty assignment unless recency-of experience qualification requirements are met. ☐ Interviewed responsible manager(s) in flight operations. ☐ Examined guidance/procedures. ☐ Examined selected flight crew training/qualification records. ☐ Observed flight crew scheduling operations. ☐ Other Actions (Specify) Click or tap here to enter text.

Guidance The specification in item i) requires the pilots to operate the flight controls: PM duties do not satisfy recency-of-experience requirements for this specification. The specifications in item (i) also ensure that newly qualifying pilots have the necessary experience to operate as a required crew member in the line training qualification program. The process to ensure such pilots meet recency-of-experience requirements may be integral to the line qualification program in accordance with FLT 2.3.1. The term Pilot Monitoring (PM) has the same meaning as the term Pilot Not Flying (PNF) for the purpose of applying the specifications of this provision. according to a defined schedule in order to establish an equivalent level of recency experience. Such schedule would not have to adhere exactly to the specification in item i) a) of this provision if the level of recent experience is acceptable to the State and applicable authorities, and the PIC or SIC, as applicable, is required to operate the flight controls in order to satisfy recency-of-experience requirements. Item v) specifies that a flight crew member whose recency has lapsed for any reason becomes unqualified and must be re-qualified by the operator. The requalification program for such a flight crewmember need not specify the same number of takeoffs and landings as the recency requirements. Applicable authorities include those authorities that have jurisdiction over international operations conducted by an operator over the high seas or the territory of a state that is other than the State of the Operator. The term “abnormal” is used to describe a condition or situation (e.g. abnormal airframe vibration, abnormal landing configuration). The terms “normal” and “non-normal/emergency” typically refer to AOM checklists, procedures and/or maneuvers. The term “non-normal” includes AOM emergency checklists and/or procedures (i.e. an emergency procedure is a subset of non-normal). The terms can also be used to describe an event, situation or operation that would be addressed by normal or non-normal/emergency procedures or checklists. When used in this manner, the terms may be separated by forward slash marks (e.g. normal/non-normal/emergency). The term “emergency” used alone refers to declarations and non-AOM procedures.

### 3.4 Flight Crew Scheduling

FLT 3.4.1

The Operator shall have a means to ensure flight crew members are qualified and current prior to accepting and/or being assigned to duty. Such means shall consist of: - A requirement that prohibits flight crew members from operating an aircraft if not qualified for duty in accordance with requirements contained in Table 2.3. - A scheduling process that ensures flight crew members, prior to being assigned to duty, are qualified and current in accordance with the applicable flight crew qualification requirements contained in Table 2.3 and, if applicable, additional requirements of the State. (GM)

☐ Documented and Implemented (Conformity) ☐ Documented not Implemented (Finding) ☐ Implemented not Documented (Finding) ☐ Not Documented not Implemented (Finding) ☐ N/A

Auditor Comments Click or tap here to enter text.

Auditor Actions ☐ Identified/Assessed tracking/scheduling processes that prevent flight crew members from flight duty assignment unless currently qualified in accordance with Table 2.3 or other applicable requirements of the State. ☐ Interviewed responsible manager(s) in flight operations. ☐ Examined process for determining additional flight crew qualification requirements of the State. ☐ Examined selected flight crew duty assignment records (focus: satisfaction of applicable qualification requirements). ☐ Observed flight crew scheduling operations (focus: scheduling requires flight crew member qualification in accordance with Table 2.3 and requirements of State). ☐ Other Actions (Specify) Click or tap here to enter text.

Guidance The intent of this provision is to ensure flight crew member requirements and related scheduling processes preclude operation of an aircraft by a flight crew member that is not qualified and current in accordance with the specifications of the provision.

FLT 3.4.3

 The Operator shall have a methodology for the purpose of managing fatigue-related safety risks to ensure fatigue occurring in one flight, successive flights or accumulated over a period of time does not impair a flight crew member's alertness and ability to safely operate an aircraft or perform safety-related duties. Such methodology shall consist of flight time, flight duty period, duty period and rest period limitations that are in accordance with the applicable prescriptive fatigue management regulations of the State. (GM)

☐ Documented and Implemented (Conformity) ☐ Documented not Implemented (Finding) ☐ Implemented not Documented (Finding) ☐ Not Documented not Implemented (Finding) ☐ N/A

Auditor Comments Click or tap here to enter text.

Auditor Actions ☐ Identified/Assessed requirements/methodology for flight crew fatigue management. ☐ Identified/Assessed tracking/scheduling processes (focus: processes take into account flight time/flight duty period/duty period/rest period limitations in the duty assignment of flight crew members). ☐ Interviewed responsible manager(s) in flight operations. ☐ Interviewed selected scheduling personnel. ☐ Examined selected flight crew duty assignment records (focus: examples of application of flight crew fatigue management limitations/militations). ☐ Other Actions (Specify) Click or tap here to enter text.

Guidance The intent of this provision is to ensure an operator establishes a methodology for the management of crew member fatigue in a manner that: - Is based upon scientific principles and knowledge; - Is consistent with the prescriptive fatigue management; - Precludes fatigue from endangering safety of the flight.

### 3.7 Fuel, Weight/Mass and Balance, Flight Plans

 FLT 3.7.2

The Operator should delegate the authority to the PIC to ensure: - A flight is not commenced unless the usable fuel required in accordance with DSP 4.3.1 is on board the aircraft and is sufficient to complete the planned flight safely; - If fuel is consumed during a flight for purposes other than originally intended during pre-flight planning, such flight is not continued without a re-analysis and, if applicable, adjustment of the planned operation to ensure sufficient fuel remains to complete the flight safely. (GM) Note: Effective 1 September 2025, this Recommendation will be upgraded to a standard.

☐ Documented and Implemented (Conformity) ☐ Documented not Implemented (Observation) ☐ Implemented not Documented (Observation) ☐ Not Documented not Implemented (Observation) ☐ N/A

Auditor Comments Click or tap here to enter text.

Auditor Actions ☐ Identified/Assessed OM requirement for PIC to ensure required safe usable fuel on board prior to flight. ☐ Interviewed responsible manager(s) in flight operations. ☐ Other Actions (Specify) Click or tap here to enter text.

Guidance Refer to the IRM for the definition of Discretionary Fuel. The intent of this provision is for the PIC to have the authority to ensure sufficient fuel is on board the aircraft to commence or continue the planned flight safely, and to be able to authorize the loading of Discretionary Fuel if such fuel is required for the safe conduct of the flight and will not cause operating limits to be exceeded In a shared system of operational control, the PIC and the Flight Dispatcher/Flight Operations Officer share the responsibility to ensure operating limitations are not exceeded and sufficient fuel is on board to commence or continue the planned flight safely. The extent of the re-analysis or adjustment specified in item ii) is commensurate with the scope and complexity of the planned operation.

### 3.8 Aircraft Preflight and Airworthiness

 FLT 3.8.1

The Operator should have guidance and procedures that describe flight crew duties and responsibilities for the use and/or application of the ATL, MEL and CDL. Such guidance and procedures shall be included in the OM or in other documents that are available to the flight crew during flight preparation and accessible to the flight crew during flight, and shall address, as a minimum, PIC responsibilities for: - Determining the airworthiness status of the aircraft; - Ensuring, for each flight, a description of known or suspected defects that affect the operation of the aircraft is recorded in the ATL; - Precluding a flight from departing until any defect affecting airworthiness is processed in accordance with the MEL/CDL; - Ensuring the aircraft is operated in accordance with any applicable MEL/CDL Operational Procedure. (GM) Note: Effective 1 September 2025, this Recommendation will be upgraded to a standard.

☐ Documented and Implemented (Conformity) ☐ Documented not Implemented (Observation) ☐ Implemented not Documented (Observation) ☐ Not Documented not Implemented (Observation) ☐ N/A

Auditor Comments Click or tap here to enter text.

Auditor Actions ☐ Identified/Assessed OM guidance/procedures for flight crew use of ATL/MEL/CDL. ☐ Interviewed responsible manager(s) in flight operations. ☐ Other Actions (Specify) Click or tap here to enter text.

Guidance The intent of this provision is for the operator to have guidance that ensures the proper use and application of the ATL, MEL and CDL. Such guidance typically addresses: - Flight crew responsibilities related to a review of the ATL and the application of the MEL/CDL; - Instructions for when to reference the MEL/CDL regarding a malfunctioning system or component; - Instructions for the completion of log book entries that ensure defects are properly recorded for the purpose of remediation and processing in accordance with the MEL/CDL, as applicable; - If applicable, the fault identification codes, trouble codes or other entries that ensure defects are appropriately identified, categorized and tracked for the purposes of remediation and/or to identify chronic or repetitive unserviceable items; - Flight crew responsibilities related to the repetitive system or component checks that are required to conform to the MEL (e.g. verifying a redundant system is operable in the case of a single system failure); - Any additional guidance necessary to ensure the ATL, MEL and CDL are used and applied in accordance with operator requirements. The specifications of this provision also apply to equivalents for the MEL and CDL.

 FLT 3.8.10

If the Operator transports passengers and/or supernumeraries without cabin crew, the Operator should have procedures to ensure, prior to departure of a flight, passengers and/or supernumeraries, as applicable, have been briefed and are familiar with the location and use of safety equipment, to include: - Seat belts; - Emergency exits; - Life jackets (individual flotation devices), if required - Lifesaving rafts, if required - Oxygen masks; - Emergency equipment for collective use. (GM) Note: Effective 1 September 2025, this Recommendation will be upgraded to a standard.

☐ Documented and Implemented (Conformity) ☐ Documented not Implemented (Observation) ☐ Implemented not Documented (Observation) ☐ Not Documented not Implemented (Observation) ☐ N/A

Auditor Comments Click or tap here to enter text.

Auditor Actions ☐ Identified/Assessed OM guidance/procedures for preflight briefing for passengers/supernumeraries; orientation as to location/use of safety equipments. ☐ Interviewed responsible manager(s) in flight operations. ☐ Other Actions (Specify) Click or tap here to enter text.

Guidance The briefing related to the specification in item ii) also typically addresses any applicable requirements and restrictions for personnel seated adjacent to cabin emergency exits.

### 3.9 Ground Handling

FLT 3.9.1

If the Operator conducts passenger flights with or without cabin crew, the Operator shall have a procedure to ensure verification that: - Passenger and crew baggage in the passenger cabin is securely stowed; - If applicable, cargo packages and/or passenger items being transported in passenger seats are properly secured. (GM)

☐ Documented and Implemented (Conformity) ☐ Documented not Implemented (Finding) ☐ Implemented not Documented (Finding) ☐ Not Documented not Implemented (Finding) ☐ N/A

Auditor Comments Click or tap here to enter text.

Auditor Actions ☐ Identified/Assessed procedure for flight crew or cabin crew to verify cabin security (focus: baggage and cargo packages/passenger items are stowed or properly secured). ☐ Interviewed responsible manager(s) in flight operations. ☐ Interviewed flight crew and cabin crew if applicable. ☐ Other Actions (Specify) Click or tap here to enter text.

Guidance The intent of this provision is for an operator to have a procedure for verification by the flight crew or cabin crew that all baggage and, if applicable, cargo packages and/or passenger items being transported in passenger seats are stowed or properly secured. Some operators might transport smaller cargo packages (e.g. mail, COMAT items) secured in cabin passenger seats. Some operators might transport certain passenger items secured in cabin passenger seats. These types of items are typically large, valuable or fragile articles belonging to passengers that are not conducive to transport as checked baggage or appropriate for stowage in overhead bins/lockers (e.g. large musical instruments, certain electronic equipment, prominent trophies, works of art). Such items might thus be secured and carried in a dedicated cabin passenger seat (which might be purchased by the passenger-owner for the purpose of transporting the item).

FLT 3.9.6

If the Operator conducts flights from any airport when conditions are conducive to ground aircraft icing, the Operator shall have de-/anti-Icing policies and procedures published in the OM or in other documents that are available to the flight crew during flight preparation and accessible to the flight crew during flight. Such policies and procedures shall address any flight crew duties and responsibilities related to de-/anti-Icing and include: - Holdover Time tables; - A requirement for a member of the flight crew or qualified ground personnel to perform a visual check of the wings before takeoff, if any contamination is suspected; - A requirement that takeoff will not commence unless the critical surfaces are clear of any deposits that might adversely affect the performance and/or controllability of the aircraft; - A statement that delegates authority to the PIC to order De-/Anti-icing whenever deemed necessary. (GM) Note: The specifications of this provision are applicable to commercial and/or non-commercial operations.

☐ Documented and Implemented (Conformity) ☐ Documented not Implemented (Finding) ☐ Implemented not Documented (Finding) ☐ Not Documented not Implemented (Finding) ☐ N/A

Auditor Comments Click or tap here to enter text.

Auditor Actions ☐ Identified/Assessed OM policy/procedures for aircraft de-/anti-icing of aircraft (focus: availability/accessibility to flight crew prior to/during flight; description of flight crew authority/duties/responsibilities; statement that requires critical surfaces to be clear of ice prior to takeoff). ☐ Interviewed responsible manager(s) in flight operations. ☐ Other Actions (Specify) Click or tap here to enter text.

Guidance Refer to the IRM for the definitions of De-/Anti-icing Program and Holdover Time. The intent of this provision is to ensure flight crew members adhere to the clean aircraft concept prior to takeoff anytime there is a potential for the accretion of ice on aircraft critical surfaces during ground operations. Refer to Guidance associated with GRH 4.2.1 located in ISM Section 6 for specifications and associated guidance related to the establishment and maintenance of a De-/Anti-icing Program. Qualified ground personnel specified in item (ii) are typically used to perform a visual wing check in instances when the wings are not visible to the flight crew from the interior of the aircraft (e.g., cargo aircraft operations). The surfaces specified in item (iii) include: wings, flight controls, engine inlets, fuselage surfaces in front of engines or other areas defined in the AOM.

### 3.10 Airspace Rules

FLT 3.10.1

The Operator should require all commercial and non-commercial flights to be conducted under an IFR Flight Plan and in accordance with an IFR clearance. (GM) Note: Non-commercial flights in this ISARP does not mean ferry flights, test/maintenance check flights or training flights.

☐ Documented and Implemented (Conformity) ☐ Documented not Implemented (Observation) ☐ Implemented not Documented (Observation) ☐ Not Documented not Implemented (Observation) ☐ N/A

Auditor Comments Click or tap here to enter text.

Auditor Actions ☐ Identified/Assessed OM requirement for all flights to be conducted under IFR flight plan/in accordance with IFR clearance. ☐ Interviewed responsible manager(s) in flight operations. ☐ Other Actions (Specify) Click or tap here to enter text.

Guidance Refer to the IRM for the definition of Instrument Flight Rules (IFR) and Visual Flight Rules (VFR). The intent of this provision is for an operator to file an IFR flight plan with the appropriate ATS unit and obtain an IFR clearance in order to ensure its flights are afforded all of the air traffic services applicable to aircraft operating under IFR within controlled airspace. Such services typically include: - Maintenance of minimum separation standards; - Traffic advisory information; - Terrain or obstruction alerting; - Low altitude alerting; - Strategic route planning; - Automatic flight plan closure at airports with functioning control towers. The specifications of this provision do not preclude an operator from: - Operating certain portions of a commercial or non-commercial flight under VFR (visual flight rules); - Where possible, identifying portions of flights to be flown under VFR on the ATS flight plan (in lieu of filing a purely IFR Flight Plan); - Operating flight such as, maintenance, repositioning, training under VFR. Some operators might transport certain passenger items secured in cabin passenger seats. These types of items are typically large, valuable or fragile articles belonging to passengers that are not conducive to transport as checked baggage or appropriate for stowage in overhead bins/lockers (e.g. large musical instruments, certain electronic equipment, prominent trophies, works of art). Such items might thus be secured and carried in a dedicated cabin passenger seat (which might be purchased by the passenger-owner for the purpose of transporting the item).

FLT 3.10.4

The Operator should have guidance that addresses the use of standard radio phraseology when communicating with ATC, the acceptance and readback of ATC clearances and, when necessary, the clarification of such clearances to ensure understanding. Such guidance shall include, as a minimum: - A requirement for the use of the call sign; - A requirement for at least two flight crew members to monitor and confirm clearances to ensure a mutual (flight crew) understanding of accepted clearances under circumstances, as determined by the operator or flight crew, when a missed or misunderstood clearance could pose a safety risk to the flight; - A requirement that takeoff will not commence unless the critical surfaces are clear of any deposits that might adversely affect the performance and/or controllability of the aircraft; - A requirement to clarify clearances with ATC whenever any flight crew member is in doubt regarding the clearance or instruction received. (GM) Note: 1 September 2025, this Recommendation will be upgraded to a standard.

☐ Documented and Implemented (Conformity) ☐ Documented not Implemented (Observation) ☐ Implemented not Documented (Observation) ☐ Not Documented not Implemented (Observation) ☐ N/A

Auditor Comments Click or tap here to enter text.

Auditor Actions ☐ Identified/Assessed OM requirement/guidance for standard phraseology in communication with ATC. ☐ Interviewed responsible manager(s) in flight operations. ☐ Other Actions (Specify) Click or tap here to enter text.

Guidance The intent of this provision is for an operator to have policies and procedures that ensure: - The use of standard radio phraseology; - ATC clearances are clearly understood during times of increased operational risk. The specification in item ii) refers to situations when a missed or misunderstood clearance could pose a safety risk to the flight (e.g. inadequate terrain clearance, runway incursion, loss of separation). ATC clearances that have the potential to pose such safety risks, if misunderstood by the flight crew, typically include the following: - Heading, altitude/flight level, assigned route/waypoint changes; - Frequency changes during critical phases of flight;

### 3.11 In-flight Operations

FLT 3.11.1

The Operator shall have established procedures which address the following phases of seaplane operations: - Pre-flight inspection and pumping of aircraft floats; - Dock safety during engine start and systems checks; - Docking procedure: departing from and arriving to fixed and floating platforms, mooring buoys, and beaching, as applicable to the scope of the theater of operations; - Taxi and water handling procedures; - Glassy water take-off and landing; - Landing area inspection; - Weather minimums; - Minimum VFR altitudes; - Passenger briefing requirements, including instructions describing the use of life jackets; - Safety of passengers during embarkation, disembarkation from docks, watercraft or beaches; - If the theater of operations includes ocean or saltwater environments, awareness of best practices for corrosion mitigation and inspection. (GM)

☐ Documented and Implemented (Conformity) ☐ Documented not Implemented (Finding) ☐ Implemented not Documented (Finding) ☐ Not Documented not Implemented (Finding) ☐ N/A

Auditor Comments Click or tap here to enter text.

Auditor Actions ☐ Identified/Assessed OM procedures that address seaplane operations (focus: inclusion of specifications listed in this standard addressing phases of seaplane operation). ☐ Interviewed responsible manager(s) in flight operations. ☐ Other Actions (Specify) Click or tap here to enter text.

Guidance The intent of this provision is to ensure that the OM or other relevant documentation prescribes comprehensive procedures for pilots conducting seaplane operations. Standardization of operational procedures ensures that elements of aircraft handling and decision making unique to seaplane operations will be conducted in a safe and uniform manner by all pilots, thereby facilitating safe operation of aircraft at the dock, on the water, and during all phases of flight.

FLT 3.11.2

The Operator shall publish, in its OM or other policy material, procedures to ensure that all seaplane passengers receive a safety briefing with specific instruction related to emergency egress and the use of life jackets and exits. The briefing shall include, at minimum: - Proper use of seatbelts and shoulder harnesses, including ensuring that the lap belt buckle is facing up, and how to undo it; - Location of all exits and how to open them; - Location of all life jackets, how to don them, how to inflate them, and how to use other features such as whistles or strobes; - Emphasis on the necessity of exiting the aircraft before inflating life jackets. (GM)

☐ Documented and Implemented (Conformity) ☐ Documented not Implemented (Finding) ☐ Implemented not Documented (Finding) ☐ Not Documented not Implemented (Finding) ☐ N/A

Auditor Comments Click or tap here to enter text.

Auditor Actions ☐ Identified/Assessed OM policy/procedures for safety briefings (focus: instructions related to emergency egress from seaplanes). ☐ Interviewed responsible manager(s) in flight operations. ☐ Other Actions (Specify) Click or tap here to enter text.

Guidance The intent of this provision is to ensure that the Operator has a defined process to establish and maintain the standards and content of passenger safety briefings and the relevance thereof to the risks associated with seaplane accidents on the water. Regulators, such as TCCA and the FAA, in response to seaplane accident investigations and safety recommendations, have placed increased emphasis on the importance of seaplane egress safety briefings. This emphasis works in tandem with recommendations for passengers to wear floatation devices while onboard seaplanes, as well as egress training for pilots and airframe modifications to seaplanes to facilitate emergency egress. Refer to FLT 2.2.4, MNT 2.6.1 and MNT 2.6.2 for information pertaining to underwater egress and guidance related to egress training, and emergency exits. Expanded guidance can be found in AC 91-69A: Seaplane Safety for 14 CFR Part 91 Operators and in Aviation Safety Study 9401.

FLT 3.11.4

The Operator should ensure minimum flight altitude information applicable to all phases of a flight, including guidance that specifies when descent below any applicable prescribed minimum altitude is permissible, is made available to the flight crew along with instructions for the use of such information. Note: 1 September 2025, this Recommendation will be upgraded to a standard.

☐ Documented and Implemented (Conformity) ☐ Documented not Implemented (Observation) ☐ Implemented not Documented (Observation) ☐ Not Documented not Implemented (Observation) ☐ N/A

Auditor Comments Click or tap here to enter text.

Auditor Actions ☐ Identified/Assessed OM guidanace that specifies when descent below applicable prescribed minimum altitude is permissible. ☐ Interviewed responsible manager(s) in flight operations. ☐ Other Actions (Specify) Click or tap here to enter text.

Guidance Minimum prescribed safety altitudes typically include: - Minimum Safety Altitude (MSA); - Minimum Descent Altitude/Height (MDA/H); - Minimum En route Altitude (MEA); - Minimum Obstruction Clearance Altitude (MOCA); - Minimum Off-Route Altitude (MORA); - Minimum Vectoring Altitude (MVA); - Any other minimum altitudes prescribed by the Authority. The specification in item ii) refers to situations when a missed or misunderstood clearance could pose a safety risk to the flight (e.g. inadequate terrain clearance, runway incursion, loss of separation). ATC clearances that have the potential to pose such safety risks, if misunderstood by the flight crew, typically include the following: - Heading, altitude/flight level, assigned route/waypoint changes; - Frequency changes during critical phases of flight.

 FLT 3.11.17

The Operator should have a policy and/or procedures that require the flight crew to monitor fuel during flight to ensure a fuel quantity upon landing that is not less than final reserve fuel. Note: 1 September 2025, this Recommendation will be upgraded to a standard.

☐ Documented and Implemented (Conformity) ☐ Documented not Implemented (Observation) ☐ Implemented not Documented (Observation) ☐ Not Documented not Implemented (Observation) ☐ N/A

Auditor Comments Click or tap here to enter text.

Auditor Actions ☐ Identified/Assessed OM requirement/guidance for monitoring en route fuel to ensure landing with not less than final reserve fuel. ☐ Interviewed responsible manager(s) in flight operations. ☐ Other Actions (Specify) Click or tap here to enter text.

Guidance Refer to FLT 3.14.16 and FLT 3.14.17 for actions to be taken by the PIC in the event the final reserve minimum fuel quantity specified in DSP 4.3.12 cannot be protected in flight and preserved upon landing.

FLT 3.11.18

If the Operator conducts two pilot operations, the Operator shall have a task sharing policy and guidance that defines and addresses the division of duties related to the performance and prioritization of flight crew member operational tasks, to include, as a minimum: - A requirement and procedures for the use of checklists prior to, during and after all phases of flight, and in abnormal and emergency situations; - PM/PF duties for all phases of flight, to include normal, abnormal and emergency situations; - PM/PF actions during manual and automatic flight; - Flight and cabin crew duties during situations that require coordination, to include, as a minimum, emergency evacuation, medical emergency and incapacitated flight crew member. (GM)

☐ Documented and Implemented (Conformity) ☐ Documented not Implemented (Finding) ☐ Implemented not Documented (Finding) ☐ Not Documented not Implemented (Finding) ☐ N/A

Auditor Comments Click or tap here to enter text.

Auditor Actions ☐ Identified/Assessed OM policy/requirement/guidance for sharing/prioritization in performance of flight crew operational tasks (focus: guidance that addresses use of checklists; defines PF/PM duties/task sharing; defines flight/cabin crew duties/task sharing). ☐ Interviewed responsible manager(s) in flight operations. ☐ Other Action (Specify) Click or tap here to enter text.

Guidance The intent of this provision is to ensure flight crew duties are defined and appropriately divided, and that compliance with all applicable checklists contained in the AOM, MEL and CDL occurs in accordance with the operator's task sharing policy. Elements of task sharing are described in the following table. Task sharing is observed during most phases of flight and addresses areas such as: - Philosophy for the use of checklists; - Performance calculations; - Automated flight procedures for flight crew; - Manual flight procedures for flight crew; - Flight crew briefings; - Administrative duties at the appropriate times (such as top of descent and prior to commencing approach). Task sharing is applicable during emergency situations such as: - Rejected takeoff; - Engine failure or fire at V1; - TCAS/ACAS resolution advisory (RA) if applicable; - GPWS Alert if applicable; - Emergency descent. Task sharing is applicable during emergency situations that require coordination with the cabin crew such as: - Emergency evacuation; - Medical emergency; - Flight crew member incapacitation.. The term Pilot Monitoring (PM) has the same meaning as the term Pilot Not Flying (PNF) for the purpose of applying the specifications of this provision. The term “abnormal” is used to describe a condition or situation (e.g. abnormal airframe vibration, abnormal landing configuration). The terms “normal” and “non-normal/emergency” typically refer to AOM checklists, procedures and/or maneuvers. The term “non-normal” includes AOM emergency checklists and/or procedures (i.e. an emergency procedure is a subset of non-normal). The terms can also be used to describe an event, situation or operation that would be addressed by normal or non-normal/emergency procedures or checklists. When used in this manner, the terms may be separated by forward slash marks (e.g. normal/non-normal/emergency). The term “emergency” used alone refers to declarations and non-AOM procedures.

 FLT 3.11.46

T The Operator should provide, and require compliance with, operating limitations, as defined by the original equipment manufacturer (OEM) and established by the State of Registry for each aircraft type used in operations. Note: 1 September 2025, this Recommendation will be upgraded to a standard.

☐ Documented and Implemented (Conformity) ☐ Documented not Implemented (Observation) ☐ Implemented not Documented (Observation) ☐ Not Documented not Implemented (Observation) ☐ N/A

Auditor Comments Click or tap here to enter text.

Auditor Actions ☐ Identified/Assessed OM provision of requirement for compliance with operating limitations as defined by OEM. ☐ Interviewed responsible manager(s) in flight operations. ☐ Other Actions (Specify) Click or tap here to enter text.

FLT 3.11.57

The Operator should establish a risk assessment process to determine altitude and VFR weather minimums for its route structure and destinations. These minimums shall meet or exceed any regulatory minimums and shall take into consideration: - Prevailing weather conditions for routes and destinations; - Terrain and geography en route and at destinations; - Pilot skill, experience, and training; - a policy on the use of Special VFR clearances. (GM)

☐ Documented and Implemented (Conformity) ☐ Documented not Implemented (Observation) ☐ Implemented not Documented (Observation) ☐ Not Documented not Implemented (Observation) ☐ N/A

Auditor Comments Click or tap here to enter text.

Auditor Actions ☐ Identified/Examined OM policy/procedures/route manual or other relevant documentation (focus: VFR weather and altitude minimums). ☐ Identified/Examined risk assessment process for routes and destination water aerodromes (focus: determination of safe weather minimums and altitudes). ☐ Interviewed responsible manager(s) in flight operations. ☐ Other Action (Specify) Click or tap here to enter text.

Guidance The intent of this provision is to ensure that the Operator sets safe standards for weather and minimum en route altitudes with due consideration to the operating environment–including geography, terrain, and prevailing weather conditions–when determining the risks associated with routes and destination aerodromes. VFR flight in marginal VMC or in IMC conditions continues to be a contributing factor to seaplane CFIT accidents. Normally, having an explicit policy defining safe VFR weather minimums facilitates safe pilot decision making as well as ensuring that operational control personnel and management are accountable in the flight dispatch decision making process. These factors combined contribute to a company safety culture. Expanded guidance can be found in ICAO Annex 6 P1: 4.2.7 and 4.3.5.1. Refer to Aviation Investigation Report A08P0353 for a case study including route risk assessment recommendations.

FLT 3.11.58

The Operator shall have a stabilized approach policy with associated guidance, criteria, and procedures to ensure the conduct of stabilized approaches. Such policy shall specify: - A minimum height for stabilization not less than 200 feet AGL for approaches in VMC; - Aircraft configuration requirements specific to each aircraft type (such as flap and propellor settings; landing gear for amphibious aircraft); - Speed limitations; - Vertical speed limitations; - Power setting is appropriate to aircraft configuration; - Aircraft is aligned with landing area and only minor corrections required to maintain normal approach path. (GM)

☐ Documented and Implemented (Conformity) ☐ Documented not Implemented (Finding) ☐ Implemented not Documented (Finding) ☐ Not Documented not Implemented (Finding) ☐ N/A

Auditor Comments Click or tap here to enter text.

Auditor Actions ☐ Identified/Assessed OM policy/guidance/procedures for the conduct of a stabilized approach (focus: flight crew procedures/definition of criteria for stabilized approach). ☐ Interviewed responsible manager(s) in flight operations. ☐ Other Actions (Specify) Click or tap here to enter text.

Guidance The intent of this provision is for the operator to implement a stabilized approach policy, as well as have guidance, criteria, and procedures that ensure the maintenance of the intended lateral and vertical flight path during visual approaches without excessive maneuvering and with the aircraft configured for landing by the prescribed altitude. The specifications in this provision are defined with consideration of the unique operating conditions typical to seaplanes, which can involve en route altitudes of 500 feet or lower over water and unpublished visual approaches at uncertified water aerodromes.

FLT 3.11.59

If the Operator utilizes amphibious aircraft, the Operator shall have a stabilized approach policy with associated guidance, criteria and procedures to ensure the conduct of stabilized approaches. Such policy shall specify: - A minimum height for stabilization not less than 1000 feet AAL for approaches in IMC or not less than 500 ft. AAL for approaches in IMC as designated by the operator and/or State where a lower stabilization height is operationally required; - A minimum height for stabilization not less than 500 feet AAL for approaches in VMC; - Aircraft configuration requirements specific to each aircraft type (landing gear, wing flaps, speed brakes); - Speed and thrust limitations; - Vertical speed limitations; - Acceptable vertical and lateral displacement from the normal approach path. (GM) Note: This provision is not applicable for the seaplane operation. The provision was designed to assess the stabilized approach of amphibians while landing on land.

☐ Documented and Implemented (Conformity) ☐ Documented not Implemented (Finding) ☐ Implemented not Documented (Finding) ☐ Not Documented not Implemented (Finding) ☐ N/A

Auditor Comments Click or tap here to enter text.

Auditor Actions ☐ Identified/Assessed OM policy/guidance/procedures for the conduct of a stabilized approach (focus: flight crew procedures/definition of criteria for stabilized approach). ☐ Interviewed responsible manager(s) in flight operations. ☐ Examined selected output from FDA/FDM/FOQA program (if applicable) (focus: data that indicates status of fleet stabilized approach performance). ☐ Examined relevant safety objectives including SPIs/SPTs (focus: proactive measures in place for identifying and preventing unstabilized approaches). ☐ Other Actions (Specify) Click or tap here to enter text.

Guidance Refer to the IRM for the definition of Flight Data Analysis (FDA) Program, Stabilized Approach and Stabilization Heights. The intent of this provision is for the operator to implement a stabilized approach policy, as well as have guidance, criteria and procedures that ensure the maintenance of the intended lateral and vertical flight path during visual approaches and/or as depicted in published approach procedures without excessive maneuvering. The parameters to be considered at the 1000 ft. AAL and 500 ft. gates as well as in the definition of a stabilized approach are listed in items iii) through vi) of the provision. The specifications in item (i) permit an operator, in accordance with operational requirements approved or accepted by the Authority, to establish stabilization criteria for heights lower than 1000 ft. AAL, but no lower than 500 ft. AAL (IMC or VMC), for approaches designated by the operator and/or State where: - Lower minimum approach stabilization heights are authorized for turbo-propeller aircraft operations (e.g., 500 feet AAL on VMC/IMC approaches), and/or - Maneuvering at a lower height AAL is required to meet instrument or other charted approach constraints (e.g. RNAV/RNP approaches, circling approaches and charted visual approaches), and/or - Aircraft are required to comply with ATC speed constraints on final approach, and/or - Deviations from selected approach stabilization criteria at a height lower than 1000 feet AAL, but above 500 feet AAL, are operationally required, and the operator can demonstrate pilot adherence to its stabilized approach policy via a continually monitored, managed and active flight data analysis (FDA) program. The criteria used to conform to the specifications in item (vi) also typically address the maneuvering that may be required in accordance with a charted visual or instrument approach procedure.

FLT 3.11.60

The Operator shall have a go-around policy with associated procedures and guidance to ensure, when necessary, flight crews discontinue or go around from an approach in accordance with criteria established by the Operator. Such policy, procedures and guidance shall, as a minimum, address or define: - Management support for flight crew go-around decision making; - Criteria that require flight crews to discontinue or go around from an approach or a landing (prior to the selection of reverse thrust, if applicable); - The go-around maneuver; - Duties and responsibilities of the PF and PM. (GM)

☐ Documented and Implemented (Conformity) ☐ Documented not Implemented (Finding) ☐ Implemented not Documented (Finding) ☐ Not Documented not Implemented (Finding) ☐ N/A

Auditor Comments Click or tap here to enter text.

Auditor Actions ☐ Identified/Assessed OM policy/requirements for execution of a missed approach/go-around when approach not stabilized in accordance with established criteria (focus: flight crew guidance/procedures for execution of a missed approach/go-around). ☐ Interviewed responsible manager(s) in flight operations. ☐ Other Actions (Specify) Click or tap here to enter text.

Guidance The intent of this provision is to reduce the risk of ALAs by ensuring the flight crew will always discontinue or go around from an approach or landing (prior to the selection of reverse thrust) when a safe landing cannot be assured or a go-around is otherwise required. The specification in item (i) is intended to foster a culture that supports flight crew go-around decision making. It is typically expressed by senior management in a manner that: - Promotes the go-around as a normal procedure; - Encourages go-around preparedness and considers the risk of the go-around maneuver itself; - Empowers the PM (or the SIC) to call for a go-around at any time during approach and landing until the selection of reverse thrust; - Ensures that go-around decision making does not affect the PIC's emergency authority in the event of (impending) abnormal or emergency situations; - Does not inhibit flight crew reporting of go-around related events. The criteria referred to in item (ii), which would require a go-around or discontinuation of an approach, typically include: - The visibility or ceiling is below the minimum required for the type of approach at the specified gates (e.g. outer marker, 1,000' AAL or at minimums). - The appropriate visual references are not obtained or are lost at or below MDA (or minimum descent height) or DA (or decision height) and through flare and touchdown by either pilot. - Prior to touchdown the wind is above the operational or pre-determined wind limit, or the runway status is below the limit determined by the flight crew's landing performance assessment. - The criteria for a stable approach are not met at the relevant approach gate(s) or can no longer be maintained until touchdown. - Technical defects or failures occur during approach that might inhibit a safe continuation of approach, landing or go-around. - Doubts by either pilot about the aircraft's geographic or spatial position. - Confusion by either pilot about the use or behavior of the automation. - It is foreseeable that the go-around routing and path will not be sufficiently clear of adverse weather or restricting traffic. - If instructed by ATC. - If required for type-specific reasons as outlined in the respective AOM. - If required by special considerations associated with a CAT II/III operation. The specification in item (iii) refers to the aircraft type-specific maneuver(s) for discontinuing a visual approach, an instrument approach or a landing prior to the selection of reverse thrust (i.e. rejected landing). The specification in item (iv) typically addresses: - Timely and effective PF briefings. - PM stabilized approach criteria deviation call-outs and compliance checks. - PF and/or PM go-around call-outs and subsequent execution of the go-around maneuver. - PF/PM go-around-related memory items. - PM actions in the event of (subtle) PF incapacitation or delayed response to a go-around callout. - PF/PM actions in the event of destabilization below stabilization height including PM monitoring for possible excessive deviations from flight path, speed, vertical speed, pitch or bank during the approach, during the transition from approach to landing and during flare and touchdown. - As applicable, the role of additional flight crew members on the flight deck (e.g., augmented crew members). To support SRM activities an operator would typically: - Include and monitor aircraft parameters related to CFIT and runway excursions in their flight data analysis (FDA) program in accordance with provisions in ORG sub-section 3.3. - Monitor go-around policy compliance through their FDA program and establish go-around safety performance indicators (SPIs). In addition to monitoring go-arounds, aircraft operators would also monitor discontinued approaches. - Include unstable approaches followed by a landing as a reporting event by the flight crew. - Minimize the need for the flight crew to report a go-around due to an unstable approach unless there is another significant event associated with the go-around (e.g. flap overspeed). An operator, in accordance with requirements of the Authority and consistent with OEM guidance, typically develops a go-around policy, guidance, criteria and procedures based on one or more of the following source references: - Global Action Plan for the Prevention of Runway Excursions Coordinated by EUROCONTROL and the Flight Safety Foundation – January 2021; - Flight Safety Foundation Go-Around Decision-Making and Execution Project Final Report March 2017; - Flight Safety Foundation Reducing the Risk of Runway Excursions – Report of the Runway Safety Initiative – May 2009; - BEA Study on Aeroplane State Awareness during Go-Around – August 2013; - Any equivalent reference document approved or accepted by the Authority for the development of flight crew guidance related to the establishment of go-around policy and the prevention of unstable approaches and runway excursions.

### 3.12 Flight Deck Policy and Procedures

FLT 3.12.1

The Operator should have a corrective lenses policy that addresses the need for flight crew members, who are required to use corrective lenses, to have a spare set of corrective lenses readily available. (GM) Note: Effective 1 September 2025, this Recommendation will be upgraded to a standard.

☐ Documented and Implemented (Conformity) ☐ Documented not Implemented (Observation) ☐ Implemented not Documented (Observation) ☐ Not Documented not Implemented (Observation) ☐ N/A

Auditor Comments Click or tap here to enter text.

Auditor Actions ☐ Identified/Assessed policy/requirement for flight crew members that require use of corrective lenses to have a spare set readily available. ☐ Interviewed responsible manager(s) in flight operations. ☐ Interviewed flight crew member(s) in flight operations. ☐ Other Actions (Specify). Click or tap here to enter text.

Guidance Corrective lens requirements are typically listed on a medical certificate or license issued by the State.

 FLT 3.12.2

The Operator should have a policy that requires flight crew members to keep their seat belts fastened when at their assigned stations and: - Those flight crew members occupying a pilot's seat to keep their safety harnesses (shoulder straps and seat belts) fastened during the takeoff and landing phases of flight; - Other flight crew members to keep their safety harnesses fastened during the takeoff and landing phases of flight, unless the shoulder straps interfere with the performance of duties, in which case the shoulder straps may be unfastened but the seat belts shall remain fastened. Note: Effective 1 September 2025, this Recommendation will be upgraded to a standard.

☐ Documented and Implemented (Conformity) ☐ Documented not Implemented (Observation) ☐ Implemented not Documented (Observation) ☐ Not Documented not Implemented (Observation) ☐ N/A

Auditor Comments Click or tap here to enter text.

Auditor Actions ☐ Identified/Assessed OM policy/procedures for use of seat belts/safety harnesses when at their assigned stations. ☐ Interviewed responsible manager(s) in flight operations. ☐ Interviewed flight crew member(s) in flight operations. ☐ Other Actions (Specify). Click or tap here to enter text.

 FLT 3.12.4

The Operator should have a policy and procedures to ensure flight crew members are only permitted to leave their duty stations during flight in the performance of duties or to meet physiological needs and shall always remain on their stations during take-off and landing. Note: 1 September 2025, this Recommendation will be upgraded to a standard.

☐ Documented and Implemented (Conformity) ☐ Documented not Implemented (Observation) ☐ Implemented not Documented (Observation) ☐ Not Documented not Implemented (Observation) ☐ N/A

Auditor Comments Click or tap here to enter text.

Auditor Actions ☐ Identified/Assessed OM policy/procedures for that address flight crew members leaving duty stations during flight. ☐ Interviewed responsible manager(s) in flight operations. ☐ Interviewed flight crew member(s) in flight operations. ☐ Other Actions (Specify) Click or tap here to enter text.

Guidance The specifications of this provision do not apply to crew changes that occur in conjunction with relief and/or augmented crews.

### 3.14 Non-Normal/Abnormal and Emergency Operations

FLT 3.14.2

The Operator should have a policy that prohibits the in-flight simulation of emergencies while passengers and/or cargo are being transported on board the aircraft. Note: Effective 1 September 2025, this Recommendation will be upgraded to a standard.

☐ Documented and Implemented (Conformity) ☐ Documented not Implemented (Observation) ☐ Implemented not Documented (Observation) ☐ Not Documented not Implemented (Observation) ☐ N/A

Auditor Comments Click or tap here to enter text.

Auditor Actions ☐ Identified/Assessed OM policy that prohibits in-flight simulated emergencies with passengers/cargo on board the aircraft. ☐ Interviewed responsible manager(s) in flight operations. ☐ Interviewed flight crew member(s) in flight operations. ☐ Examined training/qualification program for instructors/evaluators/line check airmen. ☐ Other Actions (Specify). Click or tap here to enter text.

FLT 3.14.16

The Operator should have an in-flight fuel management policy that requires the PIC to advise ATC of a minimum fuel state: - When, having committed to land at a specific airport, the PIC calculates that any change to the existing clearance to that airport may result in landing with less than planned final reserve fuel; - By declaring “MINIMUM FUEL”. (GM) Note: Effective 1 September 2025, this Recommendation will be upgraded to a standard.

☐ Documented and Implemented (Conformity) ☐ Documented not Implemented (Observation) ☐ Implemented not Documented (Observation) ☐ Not Documented not Implemented (Observation) ☐ N/A

Auditor Comments Click or tap here to enter text.

Auditor Actions ☐ Identified/Assessed OM policy/procedures for in-flight fuel management. ☐ Interviewed responsible manager(s) in flight operations. ☐ Interviewed flight crew member(s) in flight operations. ☐ Other Actions (Specify). Click or tap here to enter text.

Guidance The intent of a “MINIMUM FUEL” declaration is to inform ATC that the flight has committed to land at a specific airport and any change to the existing clearance may result in landing with less than planned final reserve fuel. This is not an emergency situation, but rather an indication that an emergency situation is possible should any additional delay occur. Guidance on in-flight fuel management, including minimum fuel declarations, is contained in the ICAO Flight Planning and Fuel Management Manual (Doc 9976).

FLT 3.14.17

The Operator should have an in-flight fuel management policy that requires the PIC to declare a situation of fuel emergency: - When the calculated usable fuel predicted to be available upon landing at the nearest airport where a safe landing can be made is less than the planned final reserve fuel; - By declaring “MAYDAY, MAYDAY, MAYDAY, FUEL”. (GM) Note: 1 September 2025, this Recommendation will be upgraded to a standard.

☐ Documented and Implemented (Conformity) ☐ Documented not Implemented (Observation) ☐ Implemented not Documented (Observation) ☐ Not Documented not Implemented (Observation) ☐ N/A

Auditor Comments Click or tap here to enter text.

Auditor Actions ☐ Identified/Assessed OM policy/procedures for in-flight fuel management. ☐ Interviewed responsible manager(s) in flight operations. ☐ Interviewed flight crew member(s) in flight operations. ☐ Other Actions (Specify) Click or tap here to enter text.

Guidance The intent of this provision is to specify the last procedural step in a series of steps to ensure the safe completion of a flight. The “MAYDAY, MAYDAY, MAYDAY, FUEL” declaration provides the clearest and most urgent expression of an emergency situation brought about by insufficient usable fuel remaining to protect the planned final reserve. It communicates that immediate action must be taken by the PIC and the air traffic control authority to ensure that the aircraft can land as soon as possible. It is used when all opportunities to protect final reserve fuel have been exploited and in the judgment of the PIC, the flight will now land with less than final reserve fuel remaining in the tanks. The word fuel is used as part of the declaration simply to convey the exact nature of the emergency to ATC. Guidance on in-flight fuel management including emergency fuel declarations is contained in the ICAO Flight Planning and Fuel Management Manual (Doc 9976).

### 3.15 Flight Crew Reporting Requirements

 FLT 3.15.2

The Operator should have a policy that requires the PIC to report any hazardous flight condition to the appropriate ATC facility without delay. (GM) Note: Effective 1 September 2025, this Recommendation will be upgraded to a standard.

☐ Documented and Implemented (Conformity) ☐ Documented not Implemented (Observation) ☐ Implemented not Documented (Observation) ☐ Not Documented not Implemented (Observation) ☐ N/A

Auditor Comments Click or tap here to enter text.

Auditor Actions ☐ Identified/Assessed policy for flight crew ATC hazard reporting. ☐ Interviewed responsible manager(s) in flight operations. ☐ Interviewed selected flight crew member(s). ☐ Other Actions (Specify). Click or tap here to enter text.

Guidance The intent of this provision is to ensure hazards with the potential to pose safety risks to the reporting aircraft or flight operations are appropriately identified and reported to the applicable ATS unit as soon as possible. Such required reports are typically defined by the State or applicable authorities and may include types of hazards as described in the following table. Generic Hazard; Report Description. Meteorological Conditions; Un-forecast or severe weather, icing, wind shear, severe turbulence. Geophysical Events; Volcanic ash observed or encountered. Security Breaches; Air Piracy or other hostile acts that threaten the safety of the aircraft or its passengers. Wildlife; Birds or large animals in the vicinity of the airport. Facilities and Infrastructure; Inadequacy of navigational facilities or undesirable navigational aid performance or other irregularity in navigational or ground facilities. Aircraft Performance; Unable to accept or maintain RVSM and reason (e.g. turbulence, mountain wave, wake turbulence, etc.), loss of navigational capability. Lasers; Illumination activities, events or exposure. Dangerous Goods; Dangerous goods on board the aircraft in the case of an in-flight emergency and for the information of airport authorities.. GPS Anomalies; Locations of GPS interference/jamming. Note: Previously promulgated hazard information (e.g., via NOTAM) would not typically require additional reporting by the PIC.

FLT 3.15.3

The Operator should have a policy that assigns responsibility to the PIC for notifying the nearest authority, by the quickest available means, of any accident or serious incident resulting in injury, death, or substantial aircraft damage. (GM) Note: Effective 1 September 2025, this Recommendation will be upgraded to a standard.

☐ Documented and Implemented (Conformity) ☐ Documented not Implemented (Observation) ☐ Implemented not Documented (Observation) ☐ Not Documented not Implemented (Observation) ☐ N/A

Auditor Comments Click or tap here to enter text.

Auditor Actions ☐ Identified/Assessed policy for flight crew accident/incident reporting. ☐ Interviewed responsible manager(s) in flight operations. ☐ Interviewed selected flight crew member(s). ☐ Other Actions (Specify). Click or tap here to enter text.

Guidance The intent of this provision is to ensure the appropriate authority in the state where an event occurred and any other organization required by such state are expeditiously informed of any accident or serious incident resulting in injury, death, or substantial aircraft damage. Such authority and organization(s) are typically defined in the applicable Aeronautical Information Publication (AIP) and may refer to one or more entities including but not limited to local law enforcement agencies, emergency service providers, the Civil Aviation Authority (CAA) and related air accident branches, safety bureaus or boards (e.g., NTSB). The PIC, if able, typically reports an applicable event to the operator who then forwards it to the appropriate authority and other relevant organization(s).

 FLT 3.15.4

The Operator should have a policy that assigns responsibility to the PIC for: - Notifying the appropriate local authority without delay in the event of any emergency situation that necessitated action in violation of local regulations and/or procedures; - Submitting, if required by the state of occurrence, a report to the appropriate local authority and also to the Authority of the State of the Operator. (GM) Note: 1 September 2025, this Recommendation will be upgraded to a standard.

☐ Documented and Implemented (Conformity) ☐ Documented not Implemented (Observation) ☐ Implemented not Documented (Observation) ☐ Not Documented not Implemented (Observation) ☐ N/A

Auditor Comments Click or tap here to enter text.

Auditor Actions ☐ Identified/Assessed OM policy for flight crew emergency action reporting. ☐ Interviewed responsible manager(s) in flight operations. ☐ Interviewed selected flight crew member(s). ☐ Other Actions (Specify) Click or tap here to enter text.

Guidance The intent of item (i) is to ensure the appropriate authority, as specified in local instructions, is notified when an in-flight emergency requires the PIC to deviate from a local rule or procedure to the extent required to meet that emergency. Such notifications are typically made through the appropriate air traffic services (ATS) unit and involve a deviation from an assigned clearance or instrument procedure. The intent of item (ii) is to ensure required occurrence/incident reporting takes place in accordance with local regulations or procedures. This includes reports submitted to the ATS unit concerned for occurrences/incidents specifically related to the provision of air traffic services.

## 4 Operations Engineering Specifications

General Guidance Refer to the IRM for the definition of Operations Engineering.

### 4.1 Aircraft Performance

 FLT 4.1.1

The Operator should have a process, performed by Operations Engineering, to determine and maintain guidance, procedures and performance data in the OM, applicable to each aircraft type, for applicable departure, destination and alternate airports. Such guidance and data shall enable the flight crew to determine or compute: - Maximum structural weights (takeoff, landing); - Takeoff performance (accelerate-stop, close-in obstacles) that also ensures charting accuracy is accounted for, when necessary, in assessing takeoff performance in the event of a critical power unit failing at any point in the takeoff; - Climb performance (distant obstacles); - Landing performance (minimum landing distance, go-around). (GM) Note: Effective 1 September 2025, this Recommendation will be upgraded to a standard.

☐ Documented and Implemented (Conformity) ☐ Documented not Implemented (Observation) ☐ Implemented not Documented (Observation) ☐ Not Documented not Implemented (Observation) ☐ N/A

Auditor Comments Click or tap here to enter text.

Auditor Actions ☐ Identified/Assessed OM guidance/procedures/data for flight crew calculations of aircraft performance for takeoff/climb/landing at departure/destination/alternate airports. ☐ Interviewed responsible manager(s) in flight operations. ☐ Interviewed selected flight crew member(s). ☐ Other Actions (Specify) Click or tap here to enter text.

Guidance The specifications in this provision are related to the prevention of CFIT and runway excursions. The intent is to ensure the operator has a process or processes to obtain or determine the specified performance data for use by flight crew. Such process(s) also address the maintenance and publication of guidance, procedures, and performance data in the OM. Data may be tailored for airports of intended use (e.g. runway analysis). The specifications in items ii) and v) may necessitate the inclusion of guidance and/or patterns to be followed in case of engine failure during takeoff, approach and go-around. Tailored data is not always available for emergency alternate airports.

 FLT 4.1.2

The Operator should have a process, performed by Operations Engineering, to determine and maintain guidance, data and procedures in the OM, applicable to each aircraft type, that enable the flight crew to determine and/or compute aircraft performance for all phases of flight. Such guidance and data shall ensure the flight crew considers all relevant factors affecting aircraft performance, to include: - Aircraft weight (mass); - Operating procedures; - Pressure altitude appropriate to the airport elevation; - Temperature; - Wind; - Runway gradient (slope) if applicable; - Runway surface condition at the expected time of use if applicable; - Obstacle data; - NOTAMs (including airport NOTAMs); - As applicable, MEL/CDL information; - Aircraft configuration (wing flap setting); - Anti-ice usage and, when applicable, ice accretion; - As applicable, runway length used for aircraft alignment prior to takeoff; - As applicable, fuel freeze considerations during extended operations. (GM) Note: Effective 1 September 2025, this Recommendation will be upgraded to a standard.

☐ Documented and Implemented (Conformity) ☐ Documented not Implemented (Observation) ☐ Implemented not Documented (Observation) ☐ Not Documented not Implemented (Observation) ☐ N/A

Auditor Comments Click or tap here to enter text.

Auditor Actions ☐ Identified/Assessed OM guidance/procedures/data for flight crew calculations of aircraft performance for all phases of flight. ☐ Interviewed responsible manager(s) in flight operations. ☐ Interviewed selected flight crew member(s). ☐ Other Actions (Specify) Click or tap here to enter text.

Guidance The intent is to ensure the operator has a process or processes to obtain or determine the specified performance data for use by flight crew. Such process(s) also address the maintenance and publication of guidance, procedures, and performance data in the OM. The specification in item vii) could be defined by a specific contaminant type/depth (e.g. snow, slush, water, ice) or an equivalent braking action report. The specifications in xiii) refers to a determination of the length of the runway available, taking into account the loss, if any, of runway length due to alignment of the aircraft amphibious prior to takeoff. The specifications in xiv) apply to considerations regarding the use of standard fuel freeze temperatures, fuel temperature analysis and en route fuel temperature monitoring for the specific fuels used in operations. Such considerations allow the flight crew to determine the actual fuel freeze temperature during extended operations (e.g. polar operations) in order to prevent in-flight freezing of fuel.

### 4.3 Aircraft Systems and Equipment Specifications

FLT 4.3.1

The Operator shall ensure all turbine engine aircraft in its fleet with a maximum certificated takeoff mass in excess of 5,700 kg (12,566 lb), or authorized to carry more than nine passengers, are equipped with a ground proximity warning system (GPWS) that automatically provides a warning to the flight crew when the aircraft is in close proximity to the earth's surface and there is: - Excessive descent rate; - Excessive altitude loss after takeoff or go-around; - Unsafe terrain clearance. (GM)

☐ Documented and Implemented (Conformity) ☐ Documented not Implemented (Finding) ☐ Implemented not Documented (Finding) ☐ Not Documented not Implemented (Finding) ☐ N/A

Auditor Comments Click or tap here to enter text.

Auditor Actions ☐ Identified/Examined turbine engine aircraft type specifications/system descriptions related to GPWS. ☐ Identified/Examined method(s) used to ensure turbine engine aircraft are equipped with GPWS. ☐ Other Action (Specify) Click or tap here to enter text.

Guidance Refer to the IRM for the definition of Ground Proximity Warning System (GPWS). In addition to the above-specified warnings, it is recommended that the GPWS also provide warnings for the following conditions: - Excessive terrain closure rate; - Unsafe terrain clearance when not in the landing configuration: - Landing gear is not locked down; - Flaps are not in the landing position. - Excessive descent below the instrument glide path.

FLT 4.3.7

The Operator shall ensure all aircraft in its fleet are equipped with at least one Emergency Locator Transmitter (ELT) of any type, except as required in FLT 4.3.8. (GM)

☐ Documented and Implemented (Conformity) ☐ Documented not Implemented (Finding) ☐ Implemented not Documented (Finding) ☐ Not Documented not Implemented (Finding) ☐ N/A

Auditor Comments Click or tap here to enter text.

Auditor Actions ☐ Identified/Examined aircraft type specifications/system descriptions related to ELT. ☐ Identified/Examined method(s) used to ensure aircraft are equipped with any type of ELT. ☐ Other Action (Specify) Click or tap here to enter text.

Guidance Refer to the IRM for the definition of Emergency Locator Transmitter (ELT) which includes the definitions for the types of ELTs. The intent of this provision is to ensure all aircraft, regardless of configuration (passenger, cargo, combi), are equipped with an ELT.

FLT 4.3.8

The Operator shall ensure aircraft in its fleet, for which the individual certificate of airworthiness is first issued after 1 July 2008, are equipped with at least one automatic Emergency Locator Transmitter (ELT).

☐ Documented and Implemented (Conformity) ☐ Documented not Implemented (Finding) ☐ Implemented not Documented (Finding) ☐ Not Documented not Implemented (Finding) ☐ N/A

Auditor Comments Click or tap here to enter text.

Auditor Actions ☐ Identified/Examined aircraft type specifications/system descriptions related to automatic ELT. ☐ Identified/Examined method(s) used to ensure aircraft are equipped with automatic ELT. ☐ Other Action (Specify) Click or tap here to enter text.

FLT 4.3.9

The Operator should ensure all aircraft in its fleet are equipped with at least one automatic Emergency Locator Transmitter (ELT).

☐ Documented and Implemented (Conformity) ☐ Documented not Implemented (Observation) ☐ Implemented not Documented (Observation) ☐ Not Documented not Implemented (Observation) ☐ N/A

Auditor Comments Click or tap here to enter text.

Auditor Actions ☐ Identified/Examined aircraft type specifications/system descriptions related to automatic ELT. ☐ Identified/Examined method(s) used to ensure aircraft are equipped with automatic ELT. ☐ Other Action (Specify) Click or tap here to enter text.

FLT 4.3.10

If the Operator utilizes amphibious aircraft, the Operator shall ensure that they are equipped with a functioning landing gear advisory system capable of aurally notifying the pilot of the gear position while on approach and capable of warning the pilot of a gear malfunction. (GM)

☐ Documented and Implemented (Conformity) ☐ Documented not Implemented (Finding) ☐ Implemented not Documented (Finding) ☐ Not Documented not Implemented (Finding) ☐ N/A

Auditor Comments Click or tap here to enter text.

Auditor Actions ☐ Identified/Examined amphibious aircraft flight manual system description or Standard Type Certificate (STC) supplement related to gear advisory system. ☐ Identified/Examined method(s) used to ensure amphibious aircraft are equipped with gear advisory system. ☐ Other Action (Specify) Click or tap here to enter text.

Guidance Considering the consequence of a gear-down landing on water, retractable gear on amphibious seaplanes present an elevated risk to operators and pilots. The intent of this provision is to mitigate this risk with landing gear advisory systems that will alert the pilot to an improper selection of the gear position. Typically, the system is configured to announce the gear position and associated landing surface (for example, “Gear down for runway landing” and “Gear up for water landing”) as the aircraft is slowed to approach speed. Additionally, a “Check Gear” warning is usually incorporated to notify the pilot of a malfunction.

FLT 4.3.11

The Operator shall ensure all seaplanes are equipped with equipment necessary to facilitate mooring, anchoring, or maneuvering the seaplane on water, appropriate to its size, mass, and handling characteristics. This equipment shall include, at minimum: - An emergency anchor with sufficient rode (line) to prove 5:1 scope appropriate to the operating environment; - Fixed ropes for docking the aircraft as well as at least one line (50 feet or longer) carried onboard for securing aircraft to shore during emergency or beaching scenarios; - Float pumps; - In the case of single-engine seaplanes, at least one paddle stowed on board and accessible to the pilot. (GM)

☐ Documented and Implemented (Conformity) ☐ Documented not Implemented (Finding) ☐ Implemented not Documented (Finding) ☐ Not Documented not Implemented (Finding) ☐ N/A

Auditor Comments Click or tap here to enter text.

Auditor Actions ☐ Identified/Examined seaplane anchor and rode on all aircraft and assessed their suitability. ☐ Identified/Examined ropes and long lines on all aircraft and assessed their suitability. ☐ Identified/Examined float pumps stowed and available on all aircraft. ☐ Identified/Examined suitable paddles stowed on all single-engine seaplanes. ☐ Other Action (Specify) Click or tap here to enter text.

Guidance The intent of this provision is to ensure all seaplanes are equipped with the ropes and equipment necessary for standard docking procedures, float maintenance and inspections, as well as for handling abnormal or emergency scenarios. For expanded guidance refer to EASA Annex IV (CAT.IDE.A.285) and ICAO Annex 6 (P1 6.5.1).

FLT 4.3.12

The Operator shall ensure all seaplanes are equipped with one inflatable life preserver or inflatable personal flotation device for each person on board, which is stowed in a position easily accessible from the seat of the person for whose use it is provided. The floatation device shall meet the standards of the regulatory authority applicable to life preservers and flotation devices. (GM)

☐ Documented and Implemented (Conformity) ☐ Documented not Implemented (Finding) ☐ Implemented not Documented (Finding) ☐ Not Documented not Implemented (Finding) ☐ N/A

Auditor Comments Click or tap here to enter text.

Auditor Actions ☐ Identified/Examined location, quantity, and suitability of inflatable life preservers/devices for all seaplanes. ☐ Other Action (Specify) Click or tap here to enter text.

Guidance The intent of this provision is to ensure, in addition to complying with regulatory requirements for life preservers or flotation devices aboard seaplanes, that only inflatable devices which are worn by or easily accessible to the passengers are used. PFDs and life jackets that are non-inflatable can become a hazard during a seaplane crash. Inherently buoyant devices can hinder egress from a partially or completely submerged aircraft if they are worn by passengers, or they can become difficult to find and take from the aircraft. Inflatable PFDs do not present these risks, provided passengers are properly briefed that the device must not be inflated until they have exited the aircraft. Some regulatory authorities have proposed that passengers wear inflatable life jackets at all times while onboard seaplanes and Transport Canada Civil Aviation has now made this a mandatory requirement aboard seaplanes with a capacity of nine or fewer passengers. The FAA also suggests that operators consider establishing a policy where all occupants wear an inflatable PFD anytime the seaplane operates on or near the water. Expanded guidance can be found at TCCA CARs: 703.83, AC 91-69A: Seaplane Safety for 14 CFR Part 91 Operators, Aviation Safety Study SA9401, Air Transportation Safety Recommendation A94-07, TCCA Airworthiness Manual: 551.403, and ICAO Annex 6 (P1 6.5.1).

| Table 2.1–Onboard Library Specifications |  |  |
| --- | --- | --- |
| The following documents shall be included in the Onboard Library: |  |  |
| General Operating Information |  |  |
| (i) | General operating information to include: - A copy of the air operator certificate (AOC); - A copy of the operations specifications relevant to the aircraft; - If applicable a copy of the Article 83 bis agreement summary (including an English version); - The Operations Manual (OM). Note: Refer to the IRM for the definitions of Article 83 bis, Article 83 bis Agreement and Article 83 bis Agreement Summary. |  |
| Aircraft Operating Information |  |  |
| (ii) | Applicable operating information to include: |  |
|  | (a) | Normal and Emergency Checklists for each operating flight crew member as required by the AFM; |
|  | (b) | Performance tables or access to performance calculations via telecom systems (e.g ACARS) is acceptable, if completed with appropriate backup procedures; |
|  | (c) | Takeoff performance deviations (e.g. due to inoperative equipment or abnormal situations). |
| (iii) | Minimum Equipment List (MEL) and Configuration Deviation List (CDL); |  |
| (iv) | Aircraft specific weight/mass and balance instructions/data (including loadsheet); |  |
| Areas, Routes and Airport Information |  |  |
| (v) | Flight Plans (OFP and ATS) for each flight; |  |
| (vi) | The applicable departure, navigation and approach charts for use by each operating flight crew member as required by AFM; |  |
| (vii) | Route and airport instructions and information (flight crew member route guide) for each flight to include, as minimum: |  |
|  | (a) | Departure airport; |
|  | (b) | Destination airport; |
|  | (c) | En route alternate airports; |
|  | (d) | Emergency airports. |
| (viii) | If applicable, the escape routes used in case of decompression or engine failure in an area of high terrain. |  |
| Other Information |  |  |
| (ix) | Cabin safety and emergency procedures relevant to the flight crew. |  |
| (x) | Dangerous Goods manual or parts relevant to the flight crew, to include information and instructions on the carriage of dangerous goods and action to be taken in the event of an emergency. |  |
| (xi) | Security Manual or parts relevant to the flight crew, including bomb search procedures. |  |
| (xii) | Ground Handling Manual or parts relevant to the flight crew, if required for flight crew to accomplish assigned duties (recommendation only and only applicable to cargo aircraft operations). |  |

| Table 2.2–Operations Manual (OM) Content Specifications |  |  |  |
| --- | --- | --- | --- |
| This table contains the fundamental OM content specifications required to achieve conformance with FLT 1.7.1 and FLT 2.1.10. The table also specifies Section 3 (DSP) provisions that must be addressed in the sections of the OM relevant to flight crew. Note: Specific flight crew policies, guidance, data and/or procedures that must also be addressed in the sections of the OM relevant to flight crew can be found in individual Section 2 provisions and are not duplicated in the table. |  |  |  |
| General Information |  |  | DSP ISARP |
| (i) | General Operations Manual (GOM), to include: |  | None |
|  | (a) | Non-aircraft type related and/or standard operating procedures for each phase of flight, policies, procedures, checklists, descriptions, guidelines, emergency procedures and other relevant information; | None |
|  | (b) | Authorities, duties and responsibilities associated with the operational control of flights; | DSP 1.3.4, 1.3.5 |
|  | (c) | If applicable, guidance that identifies and defines the common flight documents used by the flight crew, the FOO, FOA and/or other personnel responsible for operational control. | None |
| Aircraft Operating Information |  |  | DSP ISARP |
| (ii) | Aircraft Operating Manual (AOM), to include: |  | None |
|  | (a) | Normal, abnormal/non-normal and emergency procedures, instructions and checklists; | None |
|  | (b) | Aircraft systems descriptions, limitations and performance data. | None |
| (iii) | Minimum Equipment List (MEL) and Configuration Deviation List (CDL); |  | None |
| (iv) | Aircraft specific weight/mass and balance instructions/data (including loadsheet); |  | None |
| (v) | Instructions for the computation of the quantities of fuel and oil (if required) to be carried. |  | DSP 4.3.1 |
| Areas, Routes and Airport Information |  |  |  |
| (vi) | Route and airport instructions and information (departure, destination, en route and destination alternates, to include: |  |  |
|  | (a) | Airway manuals and charts, including information regarding communication facilities and navigation aids; |  |
|  | (b) | En route VFR communication procedures and communication failure procedures; |  |
|  | (c) | VFR charts and operator route information; |  |
|  | (d) | En route navigation procedures for utilizing GPS or other navigational aids and equipment; |  |
|  | (e) | Airport and runway analysis manual or documents; |  |
|  | (f) | Charts for certified water aerodromes; |  |
|  | (g) | Information or charts for uncertified water aerodromes, including coordinates, VFR arrival and departure procedures, preferred landing areas, protected areas, docking or mooring areas, obstacles and hazards, facilities, fuel availability, emergency services and communications; |  |
|  | (h) | Instructions for the selection, designation (on the OFP) and protection of departure, en route and destination alternate airports; |  |
|  | (i) | Water aerodrome classification including crew qualification criteria such as pilot training, LIFUS and experience requirements; |  |
|  | (j) | FMS or GPS database; |  |
|  | (k) | Weather minima required at departure, en route, and at destination; |  |
|  | (l) | Minimum en route altitudes; |  |
|  | (m) | If applicable, flight monitoring requirements and instructions to ensure the PIC notifies the operator of en route flight movement or deviations from the OFP; |  |
|  | (n) | If applicable, flight planning considerations that address the continuation of a flight after the failure of the critical engine on a two-engine aircraft; |  |
|  | (o) | The essential information concerning the search and rescue services in the area over which the aircraft will be flown. |  |
| Training Information |  |  |  |
| (vii) | Training Manual, to include: |  |  |
|  | (a) | Details of all relevant training programs, policies, directives and requirements, including curricula and syllabi, as applicable, for basic operator familiarization, initial qualification, continuing qualification (including recency of experience), re-qualification, aircraft transition or conversion, upgrade to PIC and other specialized training requirements, as applicable; |  |
|  | (b) | Curricula to include: ground training, simulator training, aircraft training, evaluation and certification, line flying under supervision, and any specialized training; |  |
|  | (c) | Comprehensive syllabi to include lesson plans, procedures for training and the conduct of evaluations; |  |
|  | (d) | The training program for the development of knowledge and skills related to human performance (Crew Resource Management/Dispatch Resource Management, CRM/DRM); |  |
|  | (e) | The seaplane training specifications necessary to conform with FLT 2.2.1–FLT 2.2.4. |  |
| Other Information |  |  |  |
| (viii) | Cabin safety and emergency procedures relevant to the flight crew. |  |  |
| (ix) | Dangerous Goods manual or parts relevant to the flight crew, to include information and instructions on the carriage of dangerous goods and action to be taken in the event of an emergency. |  |  |
| (x) | Security Manual or parts relevant to the flight crew, including bomb search procedures. |  |  |
| (xi) | Ground Handling Manual or parts relevant to the flight crew, if required for flight crew to accomplish assigned duties (recommendation only and only applicable to cargo aircraft operations). |  |  |

| Table 2.3–Flight Crew Qualification Requirements |  |
| --- | --- |
| Fulfillment of the following flight crew certifications, qualifications, training and currency requirements shall be recorded and retained in accordance with ORG 2.6.1, and monitored and considered when assigning flight crew members to duty in accordance with FLT 3.4.1. |  |
| (i) | Licenses/certification, including eligibility to exercise privileges of pilot license/certificate in international operations; |
| (ii) | Specific pilot license/certification limitations (First Officer, relief pilot); |
| (iii) | Specific qualifications (LVP, RVSM, ETOPS/EDTO); |
| (iv) | Equipment qualifications (TCAS/ACAS, GPWS/EGPWS, HGS, HUD/EVS, PBN, PBCS); |
| (v) | Recency-of-experience; |
| (vi) | Medical status, including Medical Certificate; |
| (vii) | Initial training and checking/line check/proficiency check/recurrent training and checking results; |
| (viii) | Right seat qualification; |
| (ix) | Type(s) qualification; |
| (x) | Airport and route competence (including special airports); |
| (xi) | Instructor/evaluator/line check airman qualification; |
| (xii) | CRM/Human Factors training; |
| (xiii) | Dangerous goods training; |
| (xiv) | Security training; |
| (xv) | Accrued flight time, duty time, duty periods and completed rest periods for the purposes of fatigue risk management and compliance with operator or State flight and/or duty time limitations. |

Table 2.4–(Intentionally open)

| Table 2.5–Route and Airport Knowledge Requirements |  |
| --- | --- |
| Each pilot crew member, in order to conform to the specifications of FLT 2.3.1, shall have adequate knowledge of the following elements related to areas, routes or route segments, and airports to be used in operations: |  |
| (i) | Terrain and minimum safe altitudes; |
| (ii) | Seasonal meteorological conditions; |
| (iii) | Meteorological, communication and air traffic facilities, services and procedures; |
| (iv) | Search and rescue services for the areas over which the aircraft will be flown; |
| (v) | Navigational facilities and procedures associated with the route along which the flight is to take place; |
| (vi) | VFR navigation procedures; |
| (vii) | GPS navigation and programming procedures; |
| (viii) | Flight planning, including fuel requirements, W&B calculations, weather minimums, daylight and grounding times, and tide considerations (if applicable); |
| (ix) | Procedures applicable to flight paths over heavily populated areas and areas of high air traffic density; |
| (x) | Water aerodrome familiarity, including obstacles, preferred take-off and landing areas, protected areas, docks, moorings, water hazards, depths, reefs, taxiing, and arrival and departure procedures; |
| (xi) | Airport obstructions, physical layout, lighting, approach aids and arrival, departure, holding and instrument approach procedures and applicable operating minima (if applicable). |

| Table 2.6–Water Aerodrome Operational Assessment |  |
| --- | --- |
| In order to conform to FLT 2.3.3 and FLT 2.3.4, Water aerodromes included in the Operator's route structure should be documented and categorized based on the following criteria: |  |
| (i) | The suitability of the take-off and landing areas, with consideration given to their dimensions and aircraft performance, and relative exposure to or protection from areas of open water and large wind fetch; |
| (ii) | Sufficient take-off and landing distance available, based on aircraft performance, to meet requirements for TORA, TODA, LDA and balked landing or engine failure considerations; |
| (iii) | Local environmental factors such as typical wind conditions and sea state (wave height or swell); |
| (iv) | Location and height of obstacles and terrain in the take-off and departure paths; |
| (v) | Obstacles or other hazards in taxi-channels, maneuvering areas, or near take-off and landing areas, such as reefs, shoals, or pilings; |
| (vi) | Tide, current and depth considerations (if applicable); |
| (vii) | The suitability and accessibility of floating or fixed docks, moorings, or beaching areas; |
| (viii) | Any other local factors such as marine traffic, water activities, or noise abatement considerations. |